## Accepted manuscript version

## Journal of Chromatography AOpen AccessVolume 1562, Pages 27 - 3510 August 2018

## DOI for the Published journal article (PJA) is 10.1016/j.chroma.2018.05.053

Multi-residue pesticide analysis in virgin olive oil by nanoflow liquid chromatography high resolution mass spectrometry

David Moreno-González1*, Jaime Alcántara-Durán1, Silvina M. Addona2, and Miriam Beneito-Cambra1

1Analytical Chemistry Research Group, Department of Physical and Analytical Chemistry, University of Jaén, 23071 Jaén Spain

2 Instituto de Ciencias Veterinarias del Litoral, Consejo Nacional de Investigaciones

Científicas y Técnicas, Santa Fe (Argentina)

*Corresponding author: Dr. David Moreno-González. Analytical Chemistry Research Group, Department of Physical and Analytical Chemistry, University of Jaén, 23071 Jaén, Spain. Tel.: (+34) 953212758; Fax: (+34) 953 212940. E-mail: dmgonzal@ujaen.es

Abstract

In this article, a nanoflow liquid chromatography system coupled to high resolution mass spectrometry (nanoflow LC/ESI Q-Orbitrap-MS) has been applied for the development of a multiresidue pesticide method for the determination of 162 multiclass pesticides in olive oil samples. Due to the relatively high lipid content of the raw QuEChERS acetonitrile extracts obtained from this type of fatty vegetable samples, a dispersive solid phase extraction (dSPE) sorbent proposed to retain both fatty acids and triglycerides, namely Enhanced Matrix Removal-Lipid (EMR-Lipid) has been implemented as additional cleanup step. The analytical performances of the proposed method were evaluated, achieving recoveries in the range 75-119% with relative standard deviations lower than 19% (n =6). The dSPE sorbent allowed the removal of most  coextracted interferences without a significant loss of analytes. Matrix effects were also evaluated, showing a negligible effect for most of the compounds tested, when a dilution factor of 50 was applied. Notably, despite the use of relatively high dilution factors (e.g. 1:50) to minimize matrix effects, the lowest concentration levels detected with this method - in the low µg kg−1 range – are well below the corresponding maximum residue levels established by the current European legislation.

Keywords: nanoflow liquid chromatography; high resolution mass spectrometry; pesticides; olive oil; QuEChERS; Enhanced Matrix Removal-Lipid

1. Introduction

Olive grove is amongst the most important agri-food sectors within the Mediterranean basin []. The implementation of new technologies has triggered remarkable improvements and benefits in production and quality, but also some issues derived from the control of pests and diseases [,]. Misusing of pesticides can affect the olive tree, posing also health risks to consumers through the exceeded presence of their residues. Consequently, the European Union (EU) have established maximum residue levels (MRLs) for pesticides in olives and to olive oil through the use of processing factors which are yet to be established (Annex VI of 396/2005) [,]. These MRLs are usually in the µg kg-1 level. Consequently, the development of reliable analytical methods for the determination of pesticides in olive oils is therefore essential.

Sample treatment is undoubtedly the main challenge associated to the development of multiresidue methods for pesticides in olive oil. Due to the use of organic solvents such as acetonitrile, the obtained extracts typically contain a relatively large amount of coextracted fatty acids and tryglicerides together with low polar pigments []. Consequently, there is a need to include additional steps aiming at clearing the presence of fat in the final extract subjected to chromatography/mass spectrometry, and this, itself, constitutes a challenge, provided the wide array of pesticides and their different physicochemical properties that need to be addressed in current pesticide multiresidue methods. In order to isolate pesticides without the undesirable co-extraction of lipids, which may affect the detection system, several clean-up procedures have been developed in olive oil or other fatty samples including matrix solid-phase dispersion [,], gel permeation chromatography [,], liquid-liquid extraction [,,], freezing/low-temperature cleanup procedures [] or solid-phase extraction (SPE) [,]. Above all, the use of QuEChERS method (quick, easy, cheap, effective, rugged, and safe) has been extended as an straightforward sample preparation approach to determine multiresidue pesticides in fruits and vegetables as well as other complex matrices at trace levels [,,]. QuEChERS method consists of a microscale extraction using generally acetonitrile (MeCN) and partitioning with magnesium sulfate alone or in combination with other salts, followed by a clean-up of the organic extract by dispersive SPE (dSPE), playing a crucial role in the case of pesticides analysis in olive oil [,].  Different dSPE sorbents including PSA, C18 and graphitized carbon black have been proposed [,]. Recent efforts have focused on the use of specific dSPE sorbents targeting the selective removal of lipids, such as the use of zirconium oxide-based sorbents (Z-Sep) [,]. However, despite its good capacity to remove lipid components of the sample, this sorbent presents high capacity to retain certain non-polar pesticides, obtaining poor recoveries [20]. Recently, a dSPE sorbent namely Enhanced Matrix Removal-Lipid sorbent (EMR-Lipid) have been proposed to remove major lipid classes from sample matrix without great effect on pesticide recoveries [,]. This sorbent has been applied for the determination of pesticides in this type of matrix in combination with gas chromatography (GC) or liquid chromatography (LC) coupled with tandem mass spectrometry (MS/MS) [26,27,]. These recently reported methods exhibited matrix effects ranging from negligible to strong (e.g. suppression of signals above 50%) though. Thus, it would be necessary to improve the performance for those pesticides affected by signal suppression due to these mentioned matrix effects. In this context, the use of nano flow LC-ESI-MS allows both a reduction of matrix effects [,] and the improvement on the ionization efficiency and method sensitivity []. This enhancement in sensitivity is based on nanospray is a more effective process than pneumatically assisted electrospray. Due to droplet sizes in nanospray are much smaller than conventional electrospray at higher flow rates, obtaining better ionization efficiency [].This fact can be used to minimize matrix effects through the implementation of high dilution of the sample without compromising overall method sensitivity.

In this work, a nanoflow liquid chromatography system coupled to high resolution mass spectrometry (nanoflow LC/ESI Q-Orbitrap-MS) has been applied for the development of a multiresidue pesticide method for the determination of 162 multiclass pesticides in olive oil samples. The advantages of nano flow-LC have been combined with the use of a lipid-removal sorbent (EMR-lipid) as dSPE sorbent for QuEChERS sample treatment and a relatively high dilution factor of 1:50. The proposed method has been evaluated [] in terms of recovery rates and matrix effects.

2. Experimental section

2.1. Chemicals and reagents

HPLC-grade methanol (MeOH) and HPLC-grade MeCN were obtained from Sigma-Aldrich (Madrid Spain). EMR-Lipid sorbent was obtained from Agilent Technologies (Santa Clara, CA, USA) whereas, sodium chloride, anhydrous magnesium sulfate and formic acid were supplied by Sigma-Aldrich. Acetic acid (HOAc) were bought from J.T. Baker (Center Valley, PA, USA).

The 162 multiclass pesticides typically used in olive grove, including insecticides, herbicides, fungicides and acaricides, used were obtained from Sigma-Aldrich or Dr. Ehrenstorfer (Augsburg, Germany). Each analytical quality standard (>99% purity) was purchased from individual stock solution (ca. 500 g mL−1) were prepared in MeCN or MeOH and kept at -20 C. Then, a working solution containing the mixture of standards was prepared (5 g mL−1) in MeCN and also stored at -20 °C.

2.2. Sample treatment

Olive oil samples were purchased from the local market (Jaén, Spain). The samples were treated as described by López-Blanco et al. [] with minor modifications. In brief, 3 g of olive oil, 7 mL H2O Milli-Q water and 10 mL MeCN (1% HOAc) was weighed into 50 mL centrifuge tube and stirred by vortex for 30 s. Then, 4 g of MgSO4 and 1 g of NaCl were added and shaken by hand for 1 min. The extract was centrifuged at 5000 rpm for 5 min, supernatant was preserved for the second clean-up step. First of all, 1 g of EMR sorbent, contained in other 50 mL centrifuge tube, was activated with 5 mL of H2O Milli-Q water, shaking for 30 s, prior to use. Then, 5 mL of organic extract from sample partitioning were added, being the tube shaken by hand for 1 min and then centrifuged at 5000 rpm for 5 min. After then, 5 mL of the obtained supernatant were transferred with a Polish centrifuge tube, which contained 0.4 g of NaCl and 1.6 g of MgSO4.  Finally, it was shaken by hand for a minute and centrifuged for 5 min at 5000 rpm. The extract was filtrated through a 0.45 µm nylon syringe filter and diluted 1:15 with H2O Milli-Q water, to obtain a final extract dilution 1:50, so that the final extract  match the initial composition of the mobile phase of the chromatographic method.

2.3. Nanoflow liquid chromatography high resolution mass spectrometry

A Thermo Scientific EASY-nLC 1000 nano-LC system (Thermo Scientific, San Jose, USA) with an EASY-Spray PepMap® C18 column (75 μm x 150mm, 3μm particle size and 100Å pore) with integrated emitter (Thermo Scientific, San Jose, USA) was used. Viper zero-dead volume finger-tight fittings (Thermo Scientific, San Jose, USA) connections were used. The column was maintained at 25 °C, the injection volume was 1 μL and the flow rate was 200 nL min-1. The following linear gradient was used: 0-5 min 2% B, 5-15 min 30% B, 15-25 min 100% B, 25-28 min 100% B, 28-33 min 2% B and 33-37 min 2% B. Mobile phases A and B were Milli-Q water and MeCN respectively, both of them with 0.1% (v/v) formic acid. The nano-LC system was connected to a Thermo Q-Exactive Orbitrap mass spectrometer equipped with an Easy-Spray nano-electrospray ion source was used. The positive polarity ESI parameters used were those optimized in previous studies [29,30]. For qualitative and quantitative analysis Xcalibur 3.0 and TraceFinder 3.3 (Thermo Scientific) were used. Retention time (RT) are included in Table 1.

3. Results and discussion

3.1. Method optimization: MS and nanoLC parameters

Taking into consideration previous works, ionization source temperature and spray voltage were kept at 250 °C and 2.2 KV, respectively [29,30].Two product ions and a precursor ion -with a mass accuracy of less than 5 ppm- was established as criteria to obtain the unambiguous identification of the pesticides studied in the case of HRMS [33,]. The complete MS / AIF combination per cycle consists of a complete MS scan event, where a full scan of the C-Trap is performed on a specific range of masses, and an AIF scan event where all ions were fragmented in high energy collision dissociation cells (HCD), and analyzed in orbitrap once trapped in C-trap []. According to previous studies [], a resolution of 70000 was set to obtain, in terms of selectivity and total cycle acquisition, satisfactory results, obtaining a sweep rate of ca. 5 Hz in the FS experiment.

The AIF mode was carried out at 30V, this value was enough to obtain a fragmentation pattern for most of studied compounds (Table S1). In all cases, the precursor ion was the protonated molecule [M+H]+. Other studied parameters in positive mode were: S-lens RF level, 60; capillary temperature, 250˚C; spray voltage, 2.2 KV. The mass spectrometer acquired two experiments with a duty cycle of ca. 0.5 seconds/acquisition point. The first experiment was full-scan at a resolution of 70000 (at m/z 195), automatic gain control (AGC) target at 1∙106; maximum injection time (IT), 200 ms and scan range, 100-750 m/z. Secondly was collision induced dissociation without precursor ion isolation or all-ion fragmentation (AIF) mode at a resolution of 17500,   AGC target at 2∙105, maximum IT, 50 ms and scan range: 100-750 m/z. Based on ± 5 ppm accurate-mass extraction windows, extracted ion chromatograms were reconstructed for identification and quantitation purposes, in addition to obtaining two product ions in AIF. According to the obtained results, it can be concluded that EU requirements in terms of diagnostic ions are widely fulfilled [32]. An an example, the theoretical m/z values for the identification of selected pesticides together with typical relative mass errors (in ppm) are included in Table S1.

The Rt data for the identification of selected pesticides are included in Table 1, whereas the distribution of the masses versus Rt is shown in Figure S1. The mobile phases were Milli-Q water (A) and MeCN (B) both of them with 0.1 % formic acid. It was observed that percentages of B at the beginning higher than 2 % produced a poor retention of polar pesticides. Consequently, in order to improve the retention of these compounds, the initial gradient composition started with 2% of B. The precision of Rts was also evaluated, the RSD (%) values were calculated from 6 injections on different days (through 125 injections) in olive oil extracts spiked at 5 μg Kg-1 for each pesticide. These values were lower than 0.2% (within ± 0.1 minutes), in compliance with the current quality control guidelines in all cases [33].

3.2. Analytical performance

In order to evaluate the performance of the proposed method, different analytical features including matrix effect, lowest concentration level (LCL), recoveries and precision were evaluated in olive oil samples. External (solvent) standard and Matrix-matched calibration curves were established, these were evaluated at five levels of concentration (0.05, 0.5, 5, 50 and 500 μg Kg-1). The analytical parameters of the proposed method are given in Table 1. The determination coefficients (R2) were higher than 0.995 in all cases, showing that the linearity was adequate.  To establish the LOQs of the proposed method, the S/N criterion was avoided. High-resolution data from Orbitrap provides higher S/N due to negligible chemical background in extracted ion chromatograms with an m/z window of ± 5 ppm. So, the data from the lowest concentration level tested (Table 1) yielding S/N ratios distinctly higher than 10 were employed, thus, providing an insight into the actual sensitivity displayed by the method. As could be observed in Table 1, lowest concentration level tested were ranged from 0.05 to 50 µg Kg-1, being lower than their corresponding MRLs [4,5]. As an example, the extracted ion chromatograms of selected compounds at 0.05 µg kg-1 are shown in Figure 1.

The trueness of the proposed method was estimated by recovery studies in olive oil samples. Olive oil samples were spiked at 5 µg kg-1 (n = 6). The absolute recoveries were calculated by comparing concentration of studied compounds in each sample spiked before the sample treatment procedure with concentration in extracts of each sample spiked after extraction and dilution step. As shown in Table 1, recovery rates ranged between 71% and 119% for all analytes with satisfactory precision for all analytes, being in agreement with current guidelines [33]. To scrutinize the results obtained and unravel the behavior of the studied pesticides during extraction, a 2D plot representing the recovery rates versus the octanol/water partition coefficient (KOW) of the analytes tested is shown in Figure 2.  In the case of propamocarb, aldicarb, aldicarb sulfoxide, quinmerac the low recoveries (around 70 %) could be attributed to its polar nature (log KOW <-1). On the other hand, several authors have suggested that the low recoveries of highly lipophilic pesticides occurs because fat is insoluble in acetonitrile and forms an additional layer in the extraction step. Thus, a partition of lipophilic pesticides between acetonitrile and fat/oil layers is produced [,]. Then, there is a tendency to obtain low recoveries for pesticides with log KOW equal to or greater than 5 such as etofenprox (log KOW 6.9, 71%).

Matrix effect is a very relevant factor the determination of these compounds by LC-ESI-MS analysis. It describes the deviation between slope of matrix-matched calibration curves and the slope of external standard calibration curves, using the following equation []:

A dilution factor of 1:15 was applied to the final extracts obtained from the sample treatment (a final dilution factor of 1:50 is obtained, considering the ratio 3g/10 mL of QuEChERS). The matrix effects (Table 1) are illustrated as a 2D plot versus retention time in Figure 3.  Soft signal suppression ([-10%]-[-20%]) was observed in the late eluting compounds (e.g. retention times > 21 min). This fact may be due to a higher concentration of co-extracted lipids. It should be noted that, 80 % of compounds displayed a negligible matrix effect ([0%]-[±10%]), while soft/minor signal variation were obtained for the rest (between [±10%]-[±20%]), with signal suppression as predominant phenomenon. Figure 4 shows the overlapped extracted ion chromatograms of selected compounds corresponding to a standard in solvent and spiked olive oil matrix at a concentration of 5 μg kg-1. As could be observed, the signal was almost the same in both cases, showing that matrix effect is negligible. Thus, no isotopically labelled internal standard were employed in the present method, due its use is required when matrix effects is sizeable. Thus, isotopically labelled internal standard was not considered.

The precision of the method in terms of repeatability (intra-day precision) and intermediate precision (inter-day precision) was evaluated for olive oil spiked at 5 µg Kg-1 with dilution factor of 50. Each sample was analyzed 6 times and injected into the nanoflow LC system. The precision expressed as %RSD of peak areas were satisfactory, obtaining %RSD lower than 20% in all cases (Table 1).

Finally, to highlight the value the analytical performance of the proposed method, a comparison with other reported methods for the determination of pesticides in olive oil samples using LC-ESI MS and EMR-lipid sorbent is shown in Table 2. The results in terms of sensitivity and recovery are similar to that reported by conventional LC-ESI-MS methods [26,28]. However, matrix effect values with the proposed method  were significantly reduced by the use of nano flow LC-ESI-MS and high dilution factors, achieving a negligible effect for  80 % of pesticides and soft for the rest. More specifically, matrix effect for several pesticides including aldicarb, fenthion, carbendazim, pymetrozine, carbaryl, fenarimol and cyromazine was reduced from medium ([±20%]-[±50%]) or strong ([±50%]) to negligible with the use of nano flow LC-ESI-MS. This fact is due to the advantages of nanospray in terms of tolerance to signal suppression as compared to conventional electrospray. Different studies have shown that the extent of matrix effects is related to the size of the droplet occurring in the Taylor cone. Thus, lower flow rates in the nL min-1 scale generates lower droplet size, and these smaller droplets contain less number of molecules (interfering species) candidates to compete for ionization [].

5. Concluding remarks

In this work, a multiresidue method using nanoflow LC MS has been proposed for the detection and quantification of 162 pesticides in olive oil sample. For accurate identification and quantification of the analytes, precise mass measurements combined with Rt information and AIF fragmentation were used. EMR-Lipid, in the clean-up step of a QuEChERS method, has been evaluated, applying a dilution factor of 50 to the sample, obtaining a negligible matrix effects for 80% of compound and soft for the other 20%. The sensitivity achieved with the proposed allow to achieve LCL lower than their MRLs established for olive oil.

Acknowledgements

The authors acknowledge funding from the Spanish Ministerio de Economía y Competitividad (MINECO) through Project Ref. CTQ-2015-71321-P, partially co-financed with FEDER funds.  D.M.G. thanks the Spanish Ministerio de Economía y Competitividad (MINECO) for a Juan de la Cierva postdoctoral contract (Ref. FJCI-2014–19573). S.M.A thanks the Argentine National Council of Scientific and Technical Research (CONICET) for granting the scholarship pertaining to the Program of Partial Financing of Stays Abroad for Assistant Researchers. The authors acknowledge Servicios Centrales de Apoyo a la Investigación (SCAI-UJAEN) from University of Jaén and Dr. Juan Castro Mármol (SCAI-UJAEN) for technical support.

Figure Captions

Figure 1. Nanoflow LC-HRMS analysis of pesticide residues in olive oil. Extracted ion chromatograms of Azoxystrobin, Bupirimate and Isoproturon in olive oil spiked at 0.05 μg kg-1.

Figure 2. 2D-Plot of the recovery rates/Log Kow in olive oil using EMR sorbent.

Figure 3. 2D-Plot of the matrix effect of the pesticides/retention time in olive oil using EMR sorbent.

Figure 4. Extracted ion chromatogram of Cyprodinil, Prochloraz, Malathion and Isoproturon corresponding to a standard in solvent (red) and spiked olive oil matrix (blue) at a concentration of 5 μg kg-1.

Table 1. Analytical performance characteristics of the proposed method for each pesticide. MRL values for olive and olive oils for each pesticide are included.

| Compound | Rt (min) | Precision |  | Recovery (n = 6) |  | LCLa (µg·kg1) | S/N | MRL Oliveb (µg·Kg1) | MRL Olive Oilc (µg·Kg1) | Matrix Effectd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Intraday (%, n = 6) | Interday (%, n = 6) | (%) | %RSD |  |  |  |  |  |
| 3-hydroxycarbofuran | 16.74 | 6 | 12 | 113 | 6.7 | 0.5 | 108 | 20 | 100 | 0 |
| Acetamiprid | 17.84 | 7 | 3 | 75 | 3.8 | 5 | 124 | 900 | 4500 | -11 |
| Alachlor | 25.24 | 5 | 7 | 120 | 8.4 | 5 | 377 | 20 | 100 | 9 |
| Aldicarb | 26.08 | 6 | 5 | 111 | 7.5 | 0.5 | 69 | 20 | 100 | -6 |
| Aldicarb sulfone | 3.36 | 18 | 9 | 75 | 8.9 | 0.05 | 268 | 20 | 100 | 7 |
| Aldicarb sulfoxide | 11.48 | 5 | 3 | 70 | 7.6 | 0.5 | 69 | 20 | 100 | 0 |
| Ametoctradin | 23.43 | 2 | 3 | 95 | 4.5 | 0.5 | 654 | 10 | 50 | -7 |
| Atrazine | 22.35 | 4 | 8 | 71 | 4.7 | 0.5 | 511 | 500 | 2500 | -9 |
| Azinphos methyl | 23.76 | 3 | 4 | 87 | 10.9 | 5 | 147 | 50 | 250 | -2 |
| Azoxystrobin | 24.06 | 7 | 5 | 107 | 5.7 | 0.05 | 230 | 10 | 50 | 6 |
| Benalaxyl | 25.46 | 3 | 6 | 83 | 8.2 | 0.05 | 165 | 50 | 250 | -5 |
| Bifenazate | 24.51 | 3 | 4 | 107 | 8.1 | 0.5 | 799 | 20 | 100 | 0 |
| Bitertanol | 12.80 | 6 | 7 | 97 | 8.1 | 5 | 52 | 20 | 100 | -14 |
| Boscalid | 24.09 | 8 | 10 | 98 | 7.5 | 5 | 174 | 10 | 50 | -9 |
| Bromuconazole 1 | 23.91 | 6 | 6 | 100 | 14.5 | 50 | 341 | 50 | 250 | -9 |
| Bromuconazole 2 | 24.42 | 8 | 10 | 101 | 13.4 | 50 | 189 | 50 | 250 | -9 |
| Bupirimate | 22.62 | 3 | 4 | 117 | 7.5 | 0.05 | 654 | 50 | 250 | -8 |
| Buprofezin | 24.96 | 5 | 6 | 85 | 5.1 | 5 | 212 | 5000 | 25000 | -9 |
| Carbaryl | 22.00 | 5 | 6 | 107 | 8.7 | 0.5 | 22 | 20 | 100 | -7 |
| Carbendazim | 3.30 | 11 | 11 | 84 | 12.4 | 0.05 | 116 | 100 | 500 | -7 |
| Carbofuran | 21.56 | 5 | 4 | 118 | 18.6 | 0.5 | 92 | 20 | 100 | -2 |
| Carfentazone ethyl | 24.75 | 4 | 10 | 100 | 12.4 | 5 | 124 | 10 | 50 | -5 |
| Chlorantraniliprol | 23.28 | 4 | 9 | 100 | 5.1 | 0.5 | 221 | 10 | 50 | -7 |
| Chlorfenvinphos | 25.32 | 5 | 9 | 90 | 18.4 | 0.5 | 149 | 20 | 100 | 2 |
| Chlorotoluron | 22.07 | 4 | 7 | 115 | 6.6 | 0.5 | 219 | 20 | 100 | 8 |
| Chlorpyrifosmethyl | 25.89 | 6 | 11 | 117 | 6.1 | 5 | 147 | 50 | 250 | 7 |
| Clofentezin | 26.12 | 17 | 15 | 103 | 7.8 | 5 | 171 | 20 | 100 | 0 |
| Clomazone | 23.25 | 6 | 5 | 77 | 6.6 | 0.5 | 55 | 20 | 100 | -7 |
| Coumaphos | 25.99 | 2 | 8 | 107 | 6.6 | 5 | 28 | 100 | 50 | 1 |
| Cyazofamid | 25.32 | 6 | 8 | 119 | 7.7 | 5 | 360 | 20 | 100 | -17 |
| Cymoxanil | 24.07 | 4 | 8 | 108 | 14.5 | 0.5 | 264 | 50 | 250 | -3 |
| Cyproconazole | 23.64 | 4 | 6 | 88 | 6.5 | 0.05 | 36 | 50 | 250 | -10 |
| Cyprodinil | 22.84 | 10 | 13 | 86 | 6.3 | 5 | 220 | 20 | 100 | 0 |
| Cyromazine | 3.24 | 14 | 16 | 105 | 13.4 | 0.5 | 264 | 50 | 250 | -10 |
| Dazomet | 22.09 | 7 | 12 | 105 | 5.2 | 0.5 | 37 | 20 | 100 | 9 |
| DEETe | 22.32 | 6 | 8 | 107 | 5.7 | 0.5 | 71 | 10 | 50 | 3 |
| DemetonSmethylsulphon | 4.11 | 4 | 13 | 111 | 10.1 | 0.05 | 233 | 10 | 50 | -6 |
| Diazinon | 25.86 | 4 | 9 | 96 | 4.1 | 0.5 | 377 | 20 | 100 | -6 |
| Dichlorvos | 20.64 | 3 | 9 | 119 | 17.7 | 0.05 | 117 | 10 | 50 | -4 |
| Dicrotophos | 14.26 | 11 | 6 | 119 | 3.6 | 0.05 | 254 | 10 | 50 | 7 |
| Diethofencarb | 23.75 | 11 | 3 | 110 | 5.1 | 0.5 | 37 | 10 | 50 | 0 |
| Difenoconazole | 25.56 | 2 | 13 | 115 | 11.3 | 5 | 265 | 2000 | 10000 | 3 |
| Diflubenzuron | 21.68 | 7 | 12 | 105 | 18.4 | 0.5 | 185 | 50 | 250 | -9 |
| Dimethoate | 17.33 | 7 | 4 | 82 | 12.2 | 0.5 | 117 | 2000 | 10000 | -7 |
| Dimethomorph E | 23.17 | 5 | 5 | 97 | 8.1 | 0.5 | 267 | 20 | 100 | -6 |
| Dimethomorph Z | 23.39 | 5 | 7 | 96 | 12.4 | 0.5 | 60 | 20 | 100 | -16 |
| Diniconazole | 25.07 | 5 | 8 | 117 | 8.6 | 0.05 | 104 | 20 | 100 | 8 |
| Diuron | 22.54 | 19 | 8 | 75 | 6.1 | 0.05 | 796 | 20 | 100 | -16 |
| Epoxiconazole | 24.2 | 6 | 8 | 114 | 6.2 | 0.5 | 264 | 50 | 250 | -5 |
| Ethion | 26.82 | 7 | 20 | 102 | 18.6 | 5 | 147 | 10 | 50 | 1 |
| Ethirimol | 24.32 | 8 | 4 | 99 | 12.4 | 0.5 | 265 | 50 | 250 | 7 |
| Ethoprophos | 24.32 | 2 | 7 | 105 | 7.7 | 0.5 | 37 | 20 | 100 | -11 |
| Etofenprox | 25.42 | 5 | 4 | 74 | 18.4 | 0.5 | 149 | 10 | 50 | 6 |
| Fenamidone | 24.08 | 4 | 3 | 89 | 5.2 | 0.5 | 366 | 10 | 50 | 6 |
| Fenamiphos | 23.98 | 9 | 6 | 89 | 9.1 | 0.05 | 377 | 50 | 250 | -4 |
| Fenamiphos sulfone | 21.28 | 7 | 8 | 117 | 10.1 | 0.5 | 117 | 50 | 250 | -11 |
| Fenamiphos sulfoxide | 21.67 | 8 | 12 | 116 | 11.4 | 0.5 | 162 | 50 | 250 | 7 |
| Fenarimol | 23.75 | 7 | 7 | 98 | 5.9 | 5 | 796 | 20 | 100 | 4 |
| Fenazaquin | 27.33 | 5 | 14 | 74 | 4.7 | 5 | 49 | 10 | 50 | -8 |
| Fenbuconazole | 19.54 | 7 | 14 | 100 | 6.7 | 0.05 | 796 | 50 | 250 | -7 |
| Fenhexamid | 31.02 | 5 | 7 | 102 | 6.3 | 0.05 | 308 | 20 | 100 | -15 |
| Fenoxycarb | 25.5 | 5 | 10 | 107 | 12.2 | 0.05 | 25 | 3000 | 15000 | 8 |
| Fenpropathrin | 23.99 | 7 | 19 | 72 | 9.2 | 0.5 | 15 | 10 | 50 | 0 |
| Fenpropimorph | 19.19 | 5 | 12 | 79 | 8.3 | 5 | 267 | 50 | 250 | -17 |
| Fenthion | 25.70 | 5 | 4 | 92 | 6.1 | 5 | 219 | 10 | 50 | -1 |
| Fenthion sulfone | 22.97 | 9 | 4 | 99 | 7.2 | 0.5 | 215 | 10 | 50 | 7 |
| Fenthion sulfoxide | 22.97 | 4 | 6 | 95 | 6.3 | 5 | 89 | 10 | 50 | 1 |
| Fipronil | 25.29 | 2 | 4 | 98 | 8.3 | 0.5 | 99 | 10 | 25 | -6 |
| Flonicamid | 30.54 | 5 | 4 | 101 | 7.5 | 5 | 221 | 60 | 300 | -6 |
| Fluazifop | 21.99 | 4 | 4 | 100 | 7.5 | 0.5 | 610 | 10 | 50 | -3 |
| Flufenacet | 24.9 | 7 | 8 | 117 | 7.3 | 5 | 268 | 50 | 250 | -1 |
| Flufenoxuron | 27.05 | 4 | 20 | 105 | 3.7 | 50 | 366 | 50 | 250 | -1 |
| Fluopyram | 24.28 | 6 | 6 | 112 | 8.9 | 0.5 | 13 | 10 | 50 | -3 |
| Fluquinconazole | 32.01 | 2 | 23 | 85 | 7.6 | 0.05 | 52 | 50 | 250 | 5 |
| Flusilazol | 24.53 | 5 | 14 | 85 | 9.2 | 0.5 | 104 | 10 | 50 | 7 |
| Flutriafol | 22.00 | 6 | 7 | 113 | 12.4 | 0.5 | 13 | 20 | 100 | -7 |
| Formetanate | 10.32 | 5 | 11 | 78 | 5.6 | 0.5 | 308 | 10 | 50 | -15 |
| Fosthiazate | 22.04 | 8 | 5 | 111 | 14.3 | 0.5 | 254 | 20 | 100 | 8 |
| Haloxyfop | 24.67 | 6 | 10 | 99 | 14.2 | 0.5 | 212 | 10 | 50 | 0 |
| Hexaconazole | 24.75 | 5 | 14 | 77 | 9.3 | 0.05 | 48 | 20 | 100 | -1 |
| Imazalil | 18.06 | 8 | 4 | 86 | 5.9 | 0.5 | 407 | 50 | 250 | -6 |
| Imidacloprid | 16.77 | 4 | 8 | 97 | 12.7 | 0.05 | 218 | 1000 | 5000 | -6 |
| Indoxacarb | 26.16 | 3 | 7 | 100 | 6.1 | 5 | 230 | 20 | 100 | -3 |
| Iprodione | 22.97 | 5 | 5 | 119 | 7.7 | 5 | 324 | 10 | 50 | -17 |
| Iprovalicarb | 23.91 | 5 | 7 | 111 | 7.9 | 0.5 | 1102 | 20 | 100 | 2 |
| Isocarbophos | 23.34 | 7 | 4 | 90 | 13.3 | 0.5 | 876 | 10 | 50 | -14 |
| Isofenphos methyl | 25.81 | 4 | 6 | 97 | 18.4 | 0.5 | 134 | 10 | 50 | -14 |
| Isoprocarb | 22.58 | 3 | 4 | 96 | 7.2 | 0.5 | 13 | 10 | 50 | 7 |
| Isoproturon | 22.43 | 2 | 3 | 94 | 8.3 | 0.05 | 873 | 10 | 50 | -9 |
| Isoxaflutole | 23.92 | 7 | 6 | 112 | 14.6 | 0.5 | 264 | 20 | 100 | -10 |
| Kresoxim methyl | 25.36 | 5 | 6 | 111 | 7.6 | 0.5 | 57 | 200 | 1000 | 1 |
| Linuron | 23.89 | 3 | 6 | 96 | 4.4 | 5 | 101 | 50 | 250 | 9 |
| Lufenuron | 24.67 | 5 | 13 | 84 | 3.9 | 0.05 | 140 | 20 | 100 | -11 |
| Malaoxon | 21.42 | 7 | 10 | 100 | 3 | 0.5 | 233 | 20 | 100 | -4 |
| Malathion | 24.66 | 3 | 6 | 110 | 7.3 | 0.5 | 248 | 20 | 100 | -20 |
| Mandipropamid | 24.17 | 5 | 5 | 99 | 5.2 | 0.5 | 159 | 10 | 50 | 0 |
| Mepanipyrim | 24.70 | 5 | 12 | 94 | 5.5 | 0.05 | 51 | 20 | 100 | 1 |
| Metalaxyl | 22.16 | 66 | 7 | 87 | 6.3 | 0.5 | 149 | 50 | 250 | 2 |
| Metconazole | 24.9 | 6 | 12 | 106 | 6.2 | 0.05 | 363 | 50 | 250 | -9 |
| Methamidophos | 24.61 | 6 | 63 | 105 | 4.8 | 5 | 1721 | 20 | 100 | -15 |
| Methidathion | 23.7 | 4 | 5 | 112 | 6.3 | 0.5 | 97 | 20 | 100 | 3 |
| Methiocarb | 23.61 | 3 | 6 | 92 | 8.5 | 0.5 | 159 | 200 | 1000 | -2 |
| Methiocarb sulfoxide | 30.00 | 20 | 11 | 94 | 11.5 | 0.5 | 197 | 300 | 1500 | -7 |
| Methomyl | 3.27 | 5 | 5 | 86 | 4.6 | 0.5 | 46 | 50 | 250 | -12 |
| Methoxyfenozide | 24.58 | 19 | 6 | 91 | 4.1 | 0.5 | 508 | 10 | 50 | 0 |
| Metobromuron | 22.74 | 7 | 6 | 98 | 5.1 | 0.5 | 305 | 10 | 50 | -11 |
| Monocrotophos | 4.13 | 3 | 20 | 83 | 8.9 | 0.5 | 22 | 20 | 100 | -18 |
| Myclobutanil | 23.99 | 7 | 6 | 104 | 10.4 | 0.05 | 149 | 20 | 100 | -8 |
| Nitenpyram | 3.48 | 3 | 9 | 99 | 7.6 | 5 | 264 | 10 | 50 | -17 |
| Norflurazon | 22.75 | 5 | 6 | 105 | 12.8 | 0.5 | 230 | 10 | 50 | -20 |
| Omethoate | 3.98 | 3 | 8 | 100 | 1.9 | 0.5 | 200 | 10 | 50 | 2 |
| Oxadixyl | 20.41 | 8 | 7 | 75 | 7.9 | 5 | 134 | 20 | 100 | -7 |
| Oxamyl | 7.73 | 3 | 4 | 100 | 9.1 | 5 | 265 | 10 | 50 | -14 |
| Oxyfluorfen | 24.65 | 9 | 10 | 95 | 5.8 | 5 | 187 | 1000 | 5000 | 1 |
| Paclobutrazol | 23.42 | 3 | 5 | 88 | 11.5 | 0.5 | 607 | 500 | 2500 | -8 |
| Paraoxon methyl | 20.64 | 7 | 11 | 99 | 6.1 | 0.5 | 69 | 20 | 100 | 7 |
| Penconazole | 24.86 | 7 | 14 | 93 | 4.1 | 0.5 | 271 | 50 | 250 | -13 |
| Pencycuron | 25.86 | 6 | 14 | 100 | 11.1 | 0.5 | 248 | 50 | 250 | -4 |
| Pendimethalin | 22.98 | 3 | 4 | 75 | 6.1 | 0.5 | 69 | 50 | 250 | -14 |
| Penthiopyrad | 25.51 | 7 | 4 | 100 | 7.7 | 5 | 268 | 10 | 50 | 7 |
| Phenthoate | 25.71 | 7 | 15 | 100 | 6.5 | 0.5 | 149 | 20 | 100 | -17 |
| Phosalone | 26.12 | 3 | 14 | 87 | 3.6 | 5 | 305 | 20 | 100 | -9 |
| Phosmet | 23.97 | 3 | 5 | 110 | 6.3 | 0.5 | 654 | 3000 | 15000 | -9 |
| Phoxim | 26.08 | 12 | 6 | 103 | 6.9 | 0.5 | 511 | 20 | 100 | -5 |
| Pirimicarb | 4.59 | 12 | 2 | 115 | 5.1 | 0.5 | 33 | 20 | 100 | -6 |
| Pirimicarbdesmethyl | 13.36 | 7 | 9 | 100 | 8.3 | 0.05 | 230 | 20 | 100 | -10 |
| Pirimiphosmethyl | 25.40 | 7 | 10 | 110 | 18.4 | 5 | 215 | 10 | 50 | -19 |
| Prochloraz | 22.86 | 8 | 5 | 103 | 12 | 0.5 | 36 | 50 | 250 | 2 |
| Procymidone | 24.54 | 17 | 18 | 100 | 6.8 | 0.5 | 219 | 20 | 100 | -3 |
| Profenofos | 26.5 | 19 | 13 | 105 | 9.7 | 5 | 145 | 20 | 100 | 7 |
| Propamocarb | 5.07 | 4 | 13 | 74 | 7.2 | 5 | 147 | 10 | 50 | -1 |
| Propaquizafop | 26.41 | 6 | 11 | 96 | 10.3 | 5 | 873 | 50 | 250 | -2 |
| Propiconazole | 24.82 | 2 | 11 | 120 | 4.7 | 5 | 1027 | 10 | 50 | -20 |
| Propoxur | 21.38 | 10 | 9 | 98 | 3.9 | 0.5 | 55 | 50 | 250 | -12 |
| Propyzamide | 24.39 | 5 | 7 | 84 | 8.8 | 0.5 | 51 | 10 | 50 | -6 |
| Pymetrozine | 3.25 | 6 | 8 | 105 | 12.4 | 0.5 | 124 | 50 | 250 | 7 |
| Pyridate | 28.66 | 3 | 13 | 101 | 8.7 | 0.5 | 93 | 50 | 250 | 1 |
| Pyrimethanil | 21.12 | 5 | 5 | 99 | 5.6 | 5 | 525 | 20 | 100 | -7 |
| Pyriproxyfen | 23.98 | 7 | 19 | 77 | 4.5 | 5 | 4046 | 50 | 250 | -7 |
| Quinmerac | 20.42 | 5 | 14 | 80 | 5.6 | 0.5 | 230 | 100 | 500 | 7 |
| Quinoclamine | 20.72 | 9 | 12 | 96 | 17.3 | 5 | 147 | 20 | 100 | 8 |
| Quinoxyfen | 26.57 | 3 | 18 | 89 | 11.2 | 5 | 1031 | 20 | 100 | 4 |
| Rotenone | 24.96 | 7 | 12 | 114 | 6.5 | 0.5 | 39 | 20 | 100 | -1 |
| Simazine | 20.83 | 5 | 9 | 119 | 7.2 | 0.5 | 1363 | 10 | 50 | -6 |
| Spirodiclofen | 29.02 | 3 | 4 | 73 | 13.9 | 5 | 360 | 20 | 100 | 9 |
| Spirotetramat | 23.60 | 21 | 5 | 119 | 4.4 | 0.5 | 1027 | 4000 | 20000 | 7 |
| Tebuconazole | 24.48 | 2 | 7 | 119 | 12.3 | 0.5 | 573 | 50 | 250 | 8 |
| Tebufenpyrad | 26.30 | 6 | 17 | 87 | 4.1 | 5 | 324 | 50 | 250 | -7 |
| Terbuthylazine | 23.83 | 4 | 13 | 115 | 5.3 | 0.5 | 42 | 50 | 250 | -7 |
| Terbuthylazinedesethyl | 21.1 | 7 | 5 | 108 | 4.4 | 0.5 | 44 | 50 | 250 | -9 |
| TFNG | 4.44 | 3 | 10 | 103 | 9.5 | 0.5 | 360 | 60 | 300 | 9 |
| Thiabendazol | 21.00 | 4 | 15 | 112 | 13.9 | 0.5 | 19 | 50 | 250 | -19 |
| Thiacloprid | 19.86 | 4 | 6 | 93 | 8.4 | 0.5 | 174 | 4000 | 20000 | 9 |
| Thiamethoxam | 14.42 | 7 | 13 | 118 | 10.4 | 5 | 482 | 400 | 2000 | -5 |
| Thiobencarb | 21.48 | 6 | 12 | 93 | 8.5 | 0.5 | 187 | 20 | 100 | -2 |
| Thiodicarb | 26.13 | 8 | 5 | 117 | 4.9 | 0.5 | 197 | 10 | 50 | 6 |
| Triadimenol 1 | 23.31 | 5 | 4 | 100 | 7.8 | 0.5 | 221 | 100 | 500 | -13 |
| Triadimenol 2 | 23.53 | 8 | 10 | 117 | 14.2 | 0.5 | 46 | 100 | 500 | -13 |
| Triazophos | 24.87 | 7 | 8 | 118 | 7.1 | 5 | 305 | 10 | 50 | -10 |
| Trichlorfon | 33.67 | 6 | 5 | 116 | 9.3 | 0.5 | 604 | 20 | 100 | -5 |
| Triticonazole | 23.68 | 6 | 7 | 118 | 6.5 | 0.5 | 17 | 10 | 50 | -2 |
| Zoxamide | 25.53 | 3 | 7 | 103 | 5.9 | 5 | 268 | 20 | 100 | 6 |

a	Corresponds to the lowest concentration level tested giving S/N ratio equal or higher than 10.

b A MRL of 10 μg kg−1 has been also set as a default value for pesticide/commodity combinations not included in the regulation [4].

c In the case of olive oil the MRL is set by a correction factor of 5 with respect MRL established in olives for olive oil production.

d Matrix effects calculated as [(calibration curve slope in matrix/calibration curve slope in solvent)-1] x 100. Zero result means that signal or suppression or enhancement has not been observed.

e N,N-diethyl-meta-toluamide.

f4-(Trifluoromethyl)nicotinoyl glycine, N-[[4-(Trifluoromethyl)-3-pyridinyl]carbonyl]glycine.

Table 2. Comparison of the proposed method with other reported methods for the determination of pesticides in olive oil samples using LC-ESI-MS and EMR sorbent.

| Method | Precision | Matrix Effect | Recovery | Sensitivity (µg kg-1) | Number of compounds | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Nano flow LC-Q-Exactive Orbitrap | < 20% | 80% negligible 20% soft | 75-119% | LCL: 0.05-50 | 162 | Present Work |
| UHPLC-QqQ-MS/MS | < 20% | 51% negligible 30% soft 13% medium 5 % strong | 70 - 120% | LOQ: 0.1 - 91 | 67 | [26] |
| UHPLC-QqQ-MS/MS | < 20% | 53% negligible 27% soft 12% medium 8 % strong | 70 - 120% | LOQ: 10-50 | 165 | [28] |

References

[]   Handbook of Olive Oil Analysis and Properties, Ramón Aparicio, John Harwood, New York (USA), Second Edition: 2013, ©Springer Science+Business Media New York, ISBN 978-1-4614-7776-1.

[]   Production techniques in olive growing, ©International Olive Council, Madrid (Spain), First edition: May 2007, ISBN: 978-84-931663-6-6.

[] Ch. Lentza-Rizos, Insecticides authorized for use on olive tres and the relationship between their registration and residues in olive oil, Grasas y Aceites, 47 (1996) 392-396.

[] European. Commission. Regulation (EC) 396/2005 of the European Parliament and of the Council on maximum residue levels of pesticides in or on food and feed of plant and animal origin and amending Council Directive 91/414/EEC. The Official Journal of the European Union, L70 (2005), pp. 1-16

[] European Commission Commission Implementing Regulation (EU) 2015/595 of 15 April 2015 concerning a coordinated multiannual control programme of the Union for 2016, 2017 and 2018 to ensure compliance with maximum residue levels of pesticides and to assess the consumer exposure to pesticide residues in and on food of plant and animal origin. The Official Journal of the European Union, L99 (2015), pp. 7-20

[] B. Gilbert-López, J.F. García-Reyes, A. Molina-Díaz, Sample treatment and determination of pesticide residues in fatty vegetable matrices: A review, Talanta, 79 (2009) 109-128.

[] J. Zhan, J. Li, D. Liu, C. Liu, G. Yang, Z. Zhou, P. Wang, A simple method for the determination of organochlorine pollutants and the enantiomers in oil seeds based on matrix solid-phase dispersion, Food Chem. 194 (2016) 319-324.

[] B. Gilbert-López, J.F. García-Reyes, A. Lozano, A.R. Fernández-Alba, Antonio Molina-Díaz, Large-scale pesticide testing in olives by liquid chromatography-electrospray tandem mass spectrometry using two sample preparation methods based on matrix solid-phase dispersion and QuEChERS, J. Chromatogr. A, 1217 (2010) 6022-6035.

[] A.G. Sánchez, N.R. Martos, E. Ballesteros, Multiresidue analysis of pesticides in olive oil by gel permeation chromatography followed by gas chromatography-tandem mass-spectrometric determination, Anal. Chim. Acta, 558 (2006) 53-61.

[] J.L. Fernández-Moreno, F.J. Arrebola-Liébanas, A. Garrido-Frenich, J.L. Martínez-Vidal, Evaluation of different sample treatments for determining pesticide residues in fat vegetable matrices like avocado by low-pressure gas chromatography-tandem mass spectrometry, J. Chromatogr. A 1111 (2006) 97-105.

[] M.A. Farajzadeh, B. Feriduni, M.R.A. Mogaddam, Determination of triazole pesticide residues in edible oils using air-assisted liquid-liquid microextraction followed by gas chromatography with flame ionization detection, J. Sep. Sci. 38 (2015) 1002-1009.

[] A. Angioni, L. Porcu, F. Pirisi, LC/DAD/ESI/MS method for the determination of imidacloprid, thiacloprid, and spinosad in olives and olive oil after field treatment, J.  Agric. Food Chem. 59 (2011) 11359-11366.

[] R. Nortes-Méndez, J. Robles-Molina, R. López-Blanco, A. Vass, A. Molina-Díaz, J.F. Garcia-Reyes, Determination of polar pesticides in olive oil and olives by hydrophilic interaction liquid chromatography coupled to tandem mass spectrometry and high resolution mass spectrometry,

Talanta 158 (2016) 222-228.

[] L. Li, Y. Xu, C. Pan, Z. Zhou, S. Jiang, F. Liu, Simplified pesticide multiresidue analysis of soybean oil by low-temperature cleanup and dispersive solid-phase extraction coupled with gas chromatography/mass spectrometry, J. AOAC Int. 90 (2007) 1387-1393.

[] F.A. Esteve-Turrillas, A. Pastor, M. de la Guardia, Determination of pyrethroid insecticide residues in vegetable oils by using combined solid-phases extraction and tandem mass spectrometry detection, Anal. Chim. Acta, 553 (2005) 50-57.

[] S. López-Feria, S. Cárdenas, M. Valcárcel, One step carbon nanotubes-based solid-phase extraction for the gas chromatographic-mass spectrometric multiclass pesticide control in virgin olive oils, J. Chromatogr. A 1216 (2009) 7346-7350.

[] B. Socas-Rodríguez, J. González-Sálamo, A. V. Herrera-Herrera, J. Hernández-Borges, M. A. Rodríguez-Delgado, Chapter Eleven -Recent advances and developments in the QuEChERS method, Comp. Anal. Chem. 76 (2017) 319-374.

[] M. Anastassiades, S.J. Lehotay, D. Štajnbaher, F.J. Schenck, Fast and easy multiresidue method employing acetonitrile extraction/partitioning and “dispersive solid-phase extraction” for the determination of pesticide residues in produce, J. AOAC Int. 86 (2003) 412–431.

[] T. Rejczak, T. Tuzimski, A review of recent developments and trends in the QuEChERS sample preparation approach (Review), Open Chem. 13 (2015) 980-1010.

[] E. Hakme, A. Lozano, C. Ferrer, F.J. Díaz-Galiano, A.R. Fernández-Alba, Analysis of pesticide residues in olive oil and other vegetable oils, TrAC Trends  Anal. Chem. (2018) in press.

[]   J.F. García-Reyes, C. Ferrer, M.J. Gómez-Ramos, A. Molina-Díaz, A.R. Fernández-Alba, Determination of pesticide residues in olive oil and olives, TrAC Trends Anal. Chem. 26 (2007) 239-251.

[] C. Anagnostopoulos, G.E. Miliadis, Development and validation of an easy multiresidue method for the determination of multiclass pesticide residues using GC–MS/MS and LC–MS/MS in olive oil and olives, Talanta 112 (2013) 1-10.

[] L. Polgár, B. Kmellár, J.F. García-Reyes, F. Fodor, Comprehensive evaluation of the clean-up step in QuEChERS procedure for the multi-residue determination of pesticides in different vegetable oils using LC–MS/MS, Anal. Methods 4 (2012) 1142-1148.

[] T. Tuzimski, T. Rejczak, Application of HPLC-DAD after SPE/QuEChERS with ZrO2-based sorbent in d-SPE clean-up step for pesticide analysis in edible oils, Food Chem. 190 (2016) 71-79.

[] D. Moreno-González, J.F. Huertas-Pérez, A.M. García-Campaña, L. Gámiz-Gracia, Determination of carbamates in edible vegetable oils by ultra-high performance liquid chromatography-tandem mass spectrometry using a new clean-up based on zirconia for QuEChERS methodology, Talanta 128 (2014) 299-304.

[] R. López-Blanco, R. Nortes-Méndez, J. Robles-Molina, D. Moreno-González, B. Gilbert-López, J.F. García-Reyes, A. Molina-Díaz, Evaluation of different cleanup sorbents for multiresidue pesticide analysis in fatty vegetable matrices by liquid chromatography tandem mass spectrometry, J. Chromatogr. A 1456 (2016) 89-104.

[] P. Parrilla Vázquez, E. Hakme, S. Uclés, V. Cutillas, M. Martínez Galera, A.R. Mughari, A.R. Fernández-Alba, Large multiresidue analysis of pesticides in edible vegetable oils by using efficient solid-phase extraction sorbents based on quick, easy, cheap, effective, rugged and safe methodology followed by gas chromatography–tandem mass spectrometry, J. Chromatogr.A, 1463 (2016) 20-31.

[] J.V. Dias, V. Cutillas, A. Lozano, I.R. Pizzutti, A.R. Fernández-Alba, Determination of pesticides in edible oils by liquid chromatography-tandem mass spectrometry employing new generation materials for dispersive solid phase extraction clean-up, J. Chromatogr. A, 1462 (2016) 8-18.

[] D. Moreno-González, P. Pérez-Ortega, B. Gilbert-López, A. Molina-Díaz, J.F. García-Reyes and A.R. Fernández-Alba, Evaluation of nanoflow liquid chromatography high resolution mass spectrometry for pesticide residue analysis in food, J. Chromatogr. A 1512 (2017) 78-87.

[] D. Moreno-González, J. Alcántara-Durán, B. Gilbert-López, J.F. García-Reyes, A. Molina-Díaz, Matrix-effect free quantitative LC-MS analysis in complex matrices using nanoflow LC with integrated emitter tip and high dilution factors, J. Chromatogr. A 1519 (2017) 110-120.

[] Y. Shen, R. Zhao , S.J. Berger, G.A. Anderson , N. Rodriguez , R.D. Smith, High-Efficiency Nanoscale Liquid Chromatography Coupled On-Line with Mass Spectrometry Using Nanoelectrospray Ionization for Proteomics, Anal. Chem. 74 (2002) 423-4249.

[] R. Juraschek, T.  Dülcks, M. Kara, Nanoelectrospray—more than just a minimized-flow electrospray ionization source,  J. Am. Soc. Mass Spectrom. 10 (1999) 300-308.

[] European Commission (2017). Guidance document on analytical quality control and method validation procedures for pesticides residues analysis in food and feed. SANTE/11813/2017.

[] R. López-Blanco, D. Moreno-González, R. Nortes-Méndez, J.F. García-Reyes, A. Molina-Díaz, B. Gilbert-López, Experimental and theoretical determination of pesticide processing factors to model their behavior during virgin olive oil production, Food Chem. 239 (2018) 9-16.

[] European Commission decision of 12 August 2002 implementing Council Directive 96/23/EC concerning the performance of analytical methods and the interpretation of results, Commission Decision 2002/657/EEC. Off. J. European Communities, L221 (2002), 8-36.

[] P. Kumar, A. Rúbies, F. Centrich, M. Granados, N. Cortés-Francisco, J. Caixach, R. Companyó, Targeted analysis with benchtop quadrupole-orbitrap hybrid mass spectrometer: application to determination of synthetic hormones in animal urine, Anal. Chim. Acta 780 (2013) 65-73.

[] L. Rajski, M. M, Gómez-Ramos, A. R. Fernández_Alba, Large pesticide multiresidue screening method by liquid chromatography-Orbitrap mass spectrometry in full scan mode applied to fruit and vegetables. J. Chromatogr. A 1360 (2014) 119- 127.

[] S.C. Cunha,  S.J. Lehotay, K. Mastovska, J.O. Fernandes, B. M. P. P. Oliveira, Evaluation of the QuEChERS sample preparation approach for the analysis of pesticide residues in olives, J. Sep. Sci. 30 (2007) 620-632.

[] S. J. Lehotay,  K. Mastovska, S. J. Yun, Evaluation of two fast and easy methods for pesticide residue analysis in fatty food matrixes, J. AOAC Int. 88 (2005) 630-638.

[]	B.K. Matuszewski, M.L. Constanzer, C.M. Chavez-Eng, Strategies for the Assessment of Matrix Effect in Quantitative Bioanalytical Methods Based on HPLC-MS/MS, Anal. Chem. 75 (2003) 3019-3030.

[] A. Schmidt, M. Karas,  T. Dülcks, Effect of different solution flow rates on analyte ion signals in nano-ESI MS, or: when does ESI turn into nano-ESI?, J. Am. Soc. Mass Spectrom. 14 (2003) 492-500.

Presented at XVII meeting of Spanish Society of Chromatography and related techniques, held in Barcelona, Spain, 3-5 October 2017 (SECYTA 2017).