## Accepted manuscript version

## Food Chemistry Volume 245, Pages 19 – 38 25 April 2018

## DOI for the Published journal article (PJA) is 10.1016/j.foodchem.2017.10.083

Matrix-effect free multi-residue analysis of veterinary drugs in food samples of animal origin by nanoflow liquid chromatography high resolution mass spectrometry

Jaime Alcántara-Durán, David Moreno-González*, Bienvenida Gilbert-López, Antonio Molina-Díaz and Juan F. García-Reyes

Analytical Chemistry Research Group, Department of Physical and Analytical Chemistry, University of Jaén, 23071 Jaén Spain

Prepared for: Food Chemistry

Corresponding author: Dr. David Moreno-González, Analytical Chemistry Research Group, University of Jaén, 23071 Jaén, Spain. Tel.: (+34) 953218758; Fax: (+34)953 212940. E-mail: dmgonzal@ujaen.es

Abstract

In this work, a sensitive method based on nanoflow liquid chromatography high-resolution mass spectrometry has been developed for the multiresidue determination of veterinary drugs residues in honey, veal muscle, egg and milk. Salting-out supported liquid extraction was employed as sample treatment for milk, veal muscle and egg, while a modified QuEChERS procedure was used in honey. The enhancement of sensitivity provided by the nanoflow LC system also allowed the implementation of high dilution factors as high as 100:1. For all matrices tested, matrix effects were negligible starting from a dilution factor of 100, enabling, thus, the use of external standard calibration instead of matrix-matched calibration of each sample, and the subsequent increase of laboratory throughput. At spiked levels as low as 0.1 or 1 µg kg−1 before the 1:100 dilution, the obtained signals were still significantly higher than the instrumental limit of quantitation (S/N 10).

Keywords: nanoflow liquid chromatography; mass spectrometry high resolution; veterinary drugs; matrix effect

1. Introduction

The use of veterinary medicines is widely accepted in veterinary practice to treat bacterial infections of animals in livestock farming (Landers, Cohen, Wittum, & Larson, 2012). However, residues from these substances may enter the food chain at harmful levels on human health (FAO, 2003). As a result, their presence in foods of animal origin may induce the spread of drug-resistant pathogenic bacterial strains or produce allergic reactions in the human being (Florea & Nightingale, 2004). In order to protect consumers against these undesirable effects, several institutions including Food and Agriculture Organization (FAO) and the European Union (EU) have set maximum residue limits (MRLs) for some veterinary drugs in different commodities including honey, milk or eggs (FAO, 2015; European Commission, 2010). MRLs of these compounds are in the range from ng kg-1 to low g kg-1. Therefore, the development of sensitive analytical methodologies for the simultaneous multiclass determination of these compounds at such low levels is of paramount interest (Masiá, Suarez-Varela, Llopis-González & Picó, 2016).

The need of methods covering multiclass veterinary drugs can be illustrated by selected recent examples from the literature (Garrido Frenich, Romero-González, & Aguilera-Luiz, 2014). These multiresidue methods are usually based on liquid chromatographic (LC) techniques coupled to mass spectrometry detection (MS) using triple quadrupole tandem mass spectrometry (MS/MS) (Dasenaki, Michali, & Thomaidis, 2016; Geis-Asteggiante, Lehotay, & Lightfield, 2012; Robert, Gillard,& Brasseur, 2013), time-of-flight (TOF) (Ortelli, Cognard, Jan, & Edder, 2009; Peters, Bolck, & Rutgers, 2009) and Orbitrap MS (Kaufmann, Butcher, Maden, Walker, & Widmer, 2014;Wang, Leung, Chow, Chang, & Wong, 2016; Cepurnieks, Rjabova, Zacs, & Bartkevics, 2015; Gómez-Pérez, Plaza-Bolaños, Romero-González, Martínez-Vidal, & Garrido-Frenich, 2012) combined with generic sample treatments such as solid phase extraction (SPE) or Quick, Easy, Cheap, Effective, Rugged and  Safe  methodology (QuEChERS), to isolate these veterinary residues from the matrix (Pinto da Costa, Ferraz Spisso, Ulberg Pereira Alves Monteiro, Gomes Ferreira & da Nóbrega, 2015; Garrido Frenich, Aguilera-Luiz, Martinez Vidal, & Romero-Gonzalez, 2010). Unfortunately, these generic extraction procedures with limited sample clean-up stages generally allow that many coeluting interfering species may cause signal suppression of the analytes of interest. This phenomenon, commonly referred as matrix effect (Kebarle, & Tang, 1993), is the main drawback of electrospray ionization for quantitative purposes, since it compromises the accuracy and trueness of the results reducing laboratory throughput as external calibration is no longer feasible for accurate quantitation (Matuszewski, 2006). Consequently, a thorough study must be undertaken to assess matrix effects of each individual analyte/matrix combination, particularly in the case of food samples of animal origin due to their complexity. Several studies have addressed this subject with strategies to overcome, minimize or compensate matrix effect during LC-MS analysis (González-Antuña et al., 2013; Trufelli, Palma, Famiglini, &  Cappiello, 2011; Chambers, Wagrowski-Diehl, Lu, & Mazzeo, 2007).

Among the different approaches, matrix dilution is probably the simplest way to reach this goal (Stahnke, Kittlaus, Kempe, &  Alder, 2012). However, the implementation of high dilutions factors is not always feasible, as it inherently decreases the limit of detection and overall method sensitivity. Downscaling the size of liquid separations is a sound option to improve the sensitivity in LC-MS with electrospray detection (Wilm, & Mann, 1996; Desmet, & Eeltink, 2013) as a consequence of the improvement on the ionization efficiency that occurs at lower flow rates (Shen, Zhao, Berger, Anderson, Rodriguez, & Smith, 2002). In addition, nanoelectrospray ionization allows to considerably reduce matrix effects (Meher, & Chen, 2017). This fact is due to the smaller spray droplets of nanospray, which need less solvent evaporation (no auxiliary or nebulizing gases are used) and a lower number of droplet fission steps prior to ion release into the gas phase. Thus, the number of analyte molecules in the primary droplets, which may enter the MS, is increased. Consequently, the number of matrix molecules is significantly reduced in comparison to conventional electrospray (Schmidt, Karas, & Dülcks, 2003). Although these features associated to the use of nano LC-MS are well established for bioanalytical applications addressing protein or peptide analysis (Fu, Amato, Wang, McGowan,  Niedernhofer, & Wang, 2015; Wilson, Vehus, Berg,  & Lundanes, 2015), it has been long associated to robustness issues due to nanoflow liquid handling problems such as leaks and dead volumes/junctions, which have hampered their development for routine small molecule applications (Cappiello, Famiglini, Mangani, Palma, & Siviero, 2003). These topics have been overcome with the recent introduction of more robust nanoflow connections avoiding dead volumes and junctions, even with columns with integrated nano electrospray emitter tip (Kiyonami, Ravnsborg, Madsen, & Zabrouskov, 2012).

This article describes the development of a multiresidue method for the analysis of 87 veterinary drugs in honey, veal muscle, egg and milk based on nanoflow LC-HRMS. Prior to LC-MS detection, generic sample treatments such as QuEChERS (honey) and salting-out supported liquid extraction (SOSLE) (veal muscle, egg and milk) were used. The sensitivity enhancement provided by nanoflow LC and the prospects to implement a high dilution factors leading to negligible ME were studied in all the matrices evaluated.

2. Experimental section

2.1. Chemicals and Reagents

Ultrapure water (Milli-Q Gradient A10 system, Millipore Bedford, MA, USA) was used throughout the study. Ammonium sulfate, ethyl acetate and primary secondary amine (PSA) were supplied from Sigma-Aldrich (St. Louis, MO, USA). All analytical standards were obtained from Dr. Ehrenstorfer (Augsburg, Germany) or Sigma-Aldrich. Individual stock solutions of the targeted compounds (ca. 500 mg L-1 each) were prepared in methanol, acetonitrile or water and stored at -20˚C. Standards mixture solutions (10 mg L-1) were prepared by appropriate dilution of the stock solutions with methanol.

To prepare the extraction buffer (SOSLE procedure), oxalic acid (reagent plus, 99.0%), ammonium hydroxide (30 %) and ethylenediaminetetraacetic acid disodium salt (EDTA) were obtained from Sigma-Aldrich. This buffer was prepared as follows: 7.4 g EDTA and 8.6 g of oxalic acid were dissolved with approximately 0.95 L of water. Ammonium hydroxide was used to adjust pH up to 3.0. Then, the final volume was adjusted 1L with water. Oasis HLB™ SPE cartridges (200 mg, 6 mL) were purchased from Waters (Milford, MA, USA). A Supelco Visiprep™ (Bellefonte, PA, USA) SPE vacuum system, SIGMA 2-16P Sartorius Centrifuge (Newtown, Wem, Shropshire, USA) and vortex (Vortex-2 Genie, Scientific Industries) were also used. PTFE syringe filters, 0.45 µm x 13 mm (Sartorius Stedim, Goettingen, Germany) were used for filtration of the sample extracts before injection into the nanoLC system.

2.2. Veterinary drugs selected, matrices tested and sample treatment

A suite of 87 representative multiclass veterinary drugs were selected based on their widespread use and their frequency of detection in food analysis. Four different matrices of animal origin were selected including milk (high-water and fat content), veal muscle (high-protein content), egg (high-water, protein and fat content) and honey (high-sugar and low-water content). These samples were purchased in different local markets. Blank samples were checked to confirm the absence of veterinary drugs residues. These samples were employed for preparing matrix-matched calibration curves.

In the case of milk, veal muscle and egg samples SOSLE was employed (Kaufmann, Butcher, Maden, Walker, & Widmer, 2014; Wang, Leung, Chow, Chang, & Wong, 2016). The procedure is detailed in Supplementary data section. In the case of honey samples, a QuEChERS methodology was selected. The details of the QuEChERS procedure are also provided in Supplementary data section.

2.3. Nanoflow liquid chromatography high resolution mass spectrometry

Chromatographic analysis was carried out using a Thermo Scientific EASY-nLC 1000 nano-LC system (Thermo Scientific, San Jose, USA) equipped with an EASY-Spray PepMap® C18 column (75 μm x 150mm, 3μm particle size and 100Å pore) (Thermo Scientific, San Jose, USA) which included an integrated nanospray emitter. Viper zero-dead volume fingertight fittings were used (Thermo Scientific, San Jose, USA). The column was maintained at 25 °C, the injection volume was 1 μl and the flow rate was 200 nL min-1. Mobile phases A and B were milli-Q water and acetonitrile, both with 0.1% (v/v) formic acid, respectively. The following linear gradient was used: 0–7 min 8% B; 7–22 min from 8% to 27% B; 22–25 min from 27% to 95% B, 25–31 min  from 95% to 100% B; and 31–35 min 2% B.

The nanoflow UHPLC system was coupled to a Thermo Q-Exactive Orbitrap mass spectrometer, operated in the positive ionization mode and equipped with an Easy-Spray nanospray ion source housing (Thermo Scientific), where the column with the integrated nanospray was precisely mounted. The following source parameters were used: spray voltage: 2.2 kV; capillary temperature: 250˚C; S-lens RF level: 60. The acquisition comprised two different experiments with a duty cycle of ca. 0.5 seconds/acquisition point: (1) full-scan at a resolution of 70000 (measured at m/z 195) and; (2) collision induced dissociation (at a normalized collision energy of 20 (arbitrary units)) without precursor ion isolation (all-ion fragmentation (AIF) mode) at a resolution of 17500. Additional mass spectrometric settings used in full-scan experiment were as follows: automatic gain control (AGC) target: 1E6; maximum injection time (IT): 200 ms, and scan range (m/z) 170-1000. In all-ion fragmentation (AIF) experiment, AGC target was set at 2E05; maximum IT was 50 ms and the acquired m/z range was m/z 50 to 750. For identification and quantitation purposes, extracted ion chromatograms were reconstructed based on ±5 ppm accurate-mass extraction windows. TraceFinder 3.3 (Thermo Scientific) was used for qualitative and quantitative analysis. Theoretical m/z values and retention time (TR) values for the identification of selected veterinary drugs are included in Table 1.

3. Results and discussion

3.1. Mass spectrometric parameters

In order to optimize sensitivity, a study of the main ionization source variables was accomplished. Thus, source temperature was studied between 250 and 310 °C and the spray voltage was studied in the range of 2.2 to 3.1 kV. It was observed that these variables did not affect significantly the analytical response in the studied range, being the lower values finally selected.

Likewise previous studies with orbitrap mass spectrometry and low-molecular weight analysis (Rajski, Gómez-Ramos, & Fernández Alba, 2014), a resolution of 70000 was set in full-scan (ca. 5 Hz), allowing satisfactory results in terms of selectivity and total cycle acquisition. To allow a clear and unambiguous identification of the studied veterinary drugs, the European regulation establishes as stringent criterion -in the case of HRMS methods- the need of detecting both a precursor ion and one product ion with mass accuracy better than 5 ppm (European Commission, 2002; European Commission, 2015). Consequently, to fulfil this requirement, full-scan acquisition mode was combined with AIF mode. Each cycle consisted of a full MS scan event followed by an AIF scan event. The first scan event is a full scan of ions (Full MS) in the specified mass range collected in the C trap; in the second scan event (AIF), all ions are fragmented in high-energy collision dissociation cell (HCD), collected in C trap and then subjected to mass analysis in the orbitrap. The fragmentation was studied at three different HCD voltages (20, 30 and 40V). A fragmentation voltage of 20V provided at least one product ion for each analyte (Table S1, Supplementary data). Therefore, the EU requirements were met for all analytes studied (European Commission, 2002). Concerning the ionization, in most cases, the precursor ion was found to be the protonated molecule [M+H]+, except for eprinomectin, where its sodium adduct [M+Na]+ was the more abundant ion observed. As example, a mass spectra of oxybendazole in FS and AIF mode is shown in Figure 1. Accurate mass analysis of the precursor ion exhibited a relative mass error lower than 2 ppm in full-scan, while additionally product ions were obtained in AIF mode (Table S1). In most cases (Table 1), relative mass errors were below 2 ppm, without using internal calibration.

3.2. Nanoflow LC method optimization and performance

Selected chromatographic parameters were evaluated in order to obtain an appropriate separation of the analytes. Up to five different gradients were assayed by varying the total gradient time (Figure S1), using the same flow rate (200 nL min−1) and mobile phases (total time of 25, 30, 35 and 45 min, respectively). It should be highlighted that gradients in nanoflow LC are usually less steep than analytical scale methods as column equilibration takes longer. All programs started at 8% organic phase and this percentage was increased up to 100%. The increase of the gradient time to 45 min did not lead to further improvement of the separation, so the 35-min method was finally selected. Concerning the solvent composition of the sample injected, it was observed that the capacity factors (k´) of the polar analytes were lower than 1 when acetonitrile percentages higher than 8% were employed. As shown in Table 1, k´ values were higher than 1 in all cases with the selected method, when a sample composition consisting of a mixture water:acetonitrile (92:8, v/v) was employed, being in agreement with SANTE guideline (European Commission, 2015). Retention time precision was also evaluated. RSD (%) values calculated from 6 injections of milk extracts spiked at 10 µg kg-1 (dilution 1:100) in different days (across a worklist of 120 injections) are provided in Table 1. These values were lower than 0.7% in all cases, being in compliance with the current legislation (within ± 0.1 minutes) (European Commission, 2015).  The remarkable results in terms of retention time precision among the different matrix studied are also noteworthy. As an example, the extracted ion chromatograms of oxybendazole in solvent, milk, veal muscle, egg and honey with dilution factor of 100 are shown in Figure 2(A).

3.3. Optimization of sample treatment

Due to the relatively high complexity of the food samples of animal origin such as those tested, the use of a dedicated sample treatment procedure with cleanup was necessary to isolate the veterinary drugs from the rest of matrix. The wide range of polarity of the studied compounds prompted the use of a generic procedure enabling the extraction of all species in a single step. So, a relativity new sample treatment called SOSLE was tested for all samples studied (Kaufmann, Butcher, Maden, Walker, & Widmer, 2014; Wang, Leung, Chow, Chang, & Wong, 2016). This procedure combines salting-out acetonitrile/water extraction and SPE cleanup. The use of SOSLE enables the simultaneous extraction of moderately polar and polar compounds, thus, increasing sample throughput. The aqueous phase obtained in the salting-out/ water extraction step is cleaned-up by SPE. Thus, moderately polar compounds are retained into the cartridges. To elute these compounds, the organic phase extract obtained in the previous extraction/salting out step is used also as the eluting solvent collecting thus the remaining polar species. The recovery percentages were calculated to evaluate the extraction efficiency. Recoveries ranged from 70 to 120% in most cases except for tetracyclines class. For these compounds, the extraction efficiency was poor due to the strong interaction of tetracyclines with different divalent cations (Ca2+, Mg2+, Zn2+, Cu2+), leading to poor recoveries (Wei, Zhang, Xu, Gou, Li, & Fan, 2014). To release tetracyclines from their metallic complexes, the addition of a chelating agent such as EDTA or citric acid has been proposed elsewhere (Moreno-González, & García-Campaña, 2017) and was included in the final procedure. Thus, an EDTA-buffer solution, was also added to the sample before applying the SOSLE procedure, obtaining then good recoveries for all tetracyclines. We also found that the SOSLE procedure cannot be applied to honey, because the separation between organic and aqueous phase in the salting-out step was not appropriate. In this case, the use of QuEChERS was considered (Bargańska, Namieśnik,  & Ślebioda, 2011). A similar approach with tetracyclines and EDTA was also implemented. The recoveries were acceptable in all cases, as detailed in Table 2.

3.4. Analytical performance

In order to evaluate the performance of the proposed method, different parameters including sensitivity, linearity, recovery rates, precision and matrix effects were evaluated in the four representative food matrices of animal origin selected (milk, veal muscle, egg and honey). Matrix-matched (1:100 diluted extract) and external (solvent) standard calibration curves were assessed at five concentration levels ranging from 0.1 to 1000 µg kg-1. Previously, a blank (unspiked) matrix extract was also examined to ensure that none of the selected compounds were present. Regression coefficients (R2) were higher than 0.999 in all cases. The sensitivity of the method was examined with the lowest concentration levels of matrix-matched standards. First, appropriate recovery rates were obtained in most cases for analytes spiked at 10 µg kg-1 in all the matrix tested. In addition, the use of the S/N criterion (S/N > 10 for LOQ) was avoided, as the high-resolution data from orbitrap usually yield higher S/N values due to negligible chemical background in extracted ion chromatograms with an m/z window of ±5ppm. This involves extrapolate towards distinctly lower concentrations than those tested and validated. The data from the lowest concentration level tested (Table 2) yielding S/N ratios distinctly higher than 10, were used instead, thus, providing an insight into the actual sensitivity displayed by the method. As could be observed in Table 2, these values ranged between 0.1 to 1 µg kg-1, being remarkably lower than the their corresponding MRL established by the EU (European Commission, 2010).

The trueness of the proposed method was assessed by recovery studies in the different commodities. Prior to extraction, different aliquots of the homogenized samples were spiked at 10 µg kg-1 (n = 6). This concentration level was selected taking into account the MRL ranges legislated by the EU for this type of matrices (European Commission, 2010). The absolute recoveries were calculated by comparing concentration of studied compounds in each sample spiked before the sample treatment procedure with concentration in extracts of each sample spiked after extraction. The recovery rates were between 70% and 120% for all analytes with satisfactory precision for all analytes (see Table 2), fulfilling current legislation (European Commission, 2002).

To evaluate matrix effects, the slope of matrix-matched calibration curves and the slope of external standard calibration curves were compared according to the following equation: [(calibration curve slope in matrix/ calibration curve slope in solvent)-1] x 100 (Matuszewski, Constanzer, & Chavez-Eng, 2003). To strike a balance between method sensitivity, robustness and absence of matrix effects, the final extracts obtained from the sample treatment were diluted up to 40 and 100-fold with initial mobile phase. Matrix effects for all samples with a dilution factor of 100 are provided in Table 3 and the results obtained with both dilutions are illustrated in Figure S2. When a dilution factor of 40 was applied, 35% of compounds presented negligible matrix effects (< ±10% signal suppression or enhancement), while for the rest of compounds, 60% was soft (between ±10% and ±20% signal change) and for only 5% of the compounds, this effect was medium (between ±20% and ±50%).  A significant reduction in matrix effect was obtained using 1:100 dilution, which permitted negligible matrix effects for all compounds and matrices combinations assayed, being overall method sensitivity not significantly compromised. In addition, the use of matrix-matched calibration would no longer be required, improving the laboratory throughput and quantitation robustness.

The precision of the method was evaluated in terms of repeatability (intra-day precision) and intermediate precision (inter-day precision) for milk samples spiked at 10 µg kg-1.  It is worth to highlight that, this experiment could be carried out, obtaining similar results regardless the sample tested. As has been pointed out above, matrix effect was null in all samples studied, so, the precision of the method is not affected at all by the composition of the final extract. The results, expressed as %RSD of peak areas (n = 10), are shown in Table S2. Good precision (RSD lower than 8.5%) was obtained in all cases. The precision attained with the proposed approach was also evaluated across long runs of over 125 injections. In the case of ivermectin, the RSD was lower than 5% (Figure 2 (b)).

To evaluate the analytical performance of the proposed method, a comparison with other reported methods for the determination of veterinary drugs in food samples is shown in Table S3. Signals obtained with spiked standards at concentration levels as low as 0.1-1 µg kg-1, still provided signals significantly higher than the instrumental limits of quantitation (S/N 10), so, due to the use of nanoflow LC, the sensitivity of the proposed method is outstanding, achieving also negligible matrix effect for all samples studied. The high LOQs obtained in previous works could be partially attributed to the dilution required by final extracts in order to diminish matrix effects and also to the performance of the instrumentation used. So, the present work provides excellent results in terms of matrix effects without sacrificing the overall sensitivity. Repeatability was similar to that reported by conventional LC-MS methods (Aguilera-Luiz, Martínez Vidal, Romero-González, & Garrido-Frenich, 2008; Kaufmann, Butcher, Maden, Walker, & Widmer, 2014), so the poor repeatability associated with old nanoflow LC systems was not observed. In contrast, the use of this approach based on nanoflow liquid chromatography resulted advantageous. For instance, the recoveries and matrix effects obtained compared favorably against those obtained in milk by other authors using the same extraction method and the same mass spectrometer, but conventional UHPLC (Kaufmann, Butcher, Maden, Walker, & Widmer, 2014; Wang, Leung, Chow, Chang & Wong, 2015). Recoveries and matrix effects were also better than those obtained by other sample preparation protocols such as SPE or ultrafiltration (see Table S3).

Finally, to illustrate the applicability of the method, four milk samples obtained from different local market were analyzed. The veterinary drugs residues present in milk samples were quantified and identified according to the SANTE guidelines (European Commission, 2015), which establishes a tolerance level for the relative intensity between qualifier and quantifier ions. The veterinary drugs concentration was calculated from the corresponding calibration curve (Table 4). These four samples showed detectable levels of oxacillin, oxytetracycline, penicillin G, penicillin V and thiabendazole residues, although below MRLs in all cases. These substances could be expected in samples, since penicillins are antibiotics used against respiratory infections, although its use is limited in milk-producing cows, thiabendazole is a nematicide antihelmine and, oxacillin and oxytetracycline are broad-spectrum antibiotics used against bacterial, mycoplasmic and protozootic infections.

4. Concluding remarks

In this work, a multiresidue method using nanoLC-HRMS was developed for the detection and quantification of 87 veterinary drugs in food samples including honey, milk, muscle of veal and egg samples. Accurate mass measurements combined with RT information and AIF fragmentation were used for appropriate analyte quantification and identification. The use of SOSLE, for milk, veal muscle and egg, or modified QuEChERS, for honey, sample treatment permitted appropriate recovery rates in all cases. The lowest concentration level detectable of the different veterinary drugs/commodity combinations were all well below that their corresponding MRLs. Thus, the sensitivity of the proposed method was satisfactory. It is worth mentioning that, unlike in previous related studies, matrix effects were completely removed for all the compounds and matrices tested. This was accomplished by optimizing the parameters of the method and applying a dilution factor of 100 to the sample. This makes the use of matrix-matched calibration or standard addition method no longer necessary, a valuable feature considering the potential savings from its implementation in laboratories.

Acknowledgements

The authors acknowledge funding from the Spanish Ministerio de Economía y Competitividad (MINECO) through Project Ref. CTQ2015-71321-P, partially co-financed with FEDER funds. D.M.G. thanks the Spanish Ministerio de Economía y Competitividad (MINECO) for a Juan de la Cierva Formación postdoctoral contract (Ref. FJCI-2014–19573). The authors acknowledge Servicios Centrales de Apoyo a la Investigación (SCAI-UJAEN) from University of Jaén and Dr. Juan Castro Mármol (SCAI-UJAEN) for technical support.

Figure Captions

Figure 1. Mass spectrum of oxybendazole in veal spiked at 10 µg kg-1 with dilution factor of 100.

Figure 2.  (A) Extracted ion chromatograms of oxybendazole spiked at 0.1 µg L-1 in solvent and 10 µg kg -1 in milk, veal muscle, egg and honey with dilution factor of 100, and (B) Peak area precision for ivermectin at 10 µg kg-1 in milk extract with dilution factor of 100.

Figure 1

Figure 2

Table 1. List of studied veterinary drugs with chemical formula, capacity factor (k´), retention time, theoretical and experimental mass of extracted parent ion and relative mass error (expressed in parts per million (ppm)).

| Compound | Elemental Composition | Capacity Factor (k´) | RT (min) | a RSD (%) (RT (n =6)) | Theoretical m/z | Experimental m/z | Error (ppm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acetamiprid | C10H11ClN4 | 2.5 | 15.76 | 0.45 | 223.0745 | 223.0748 | 1.3 |
| Acriflavine | C14H14ClN3 | 4.4 | 20.4 | 0.55 | 260.0949 | 260.0956 | 2.7 |
| Albendazole | C12H15N3O2S | 6.9 | 20.05 | 0.29 | 226.0958 | 226.0961 | 1.3 |
| Albendazole Sulfone | C12H15N3O4S | 9.3 | 16.01 | 0.45 | 298.0856 | 298.0854 | -0.7 |
| Albendazole Sulfoxide | C12H15N3O3S | 8.3 | 10.07 | 0.49 | 282.0907 | 282.0905 | -0.7 |
| Alternogest | C21H26O2 | 7.8 | 26.32 | 0.38 | 311.2006 | 311.2014 | 2.6 |
| Amoxicillin | C16H19N3O5S | 9.2 | 1.05 | 0.41 | 336.1118 | 336.1126 | 2.4 |
| Betamethasone | C22H29FO5 | 8.3 | 22.28 | 0.42 | 393.2072 | 393.208 | 2.0 |
| Carbosulfan | C20H32N2O3S | 11.3 | 5.7 | 0.43 | 381.2206 | 381.2214 | 2.1 |
| Cefalexin | C16H17N3O4S | 9.9 | 3.33 | 0.57 | 348.1013 | 348.1021 | 2.3 |
| Cefazolin | C14H14N8O4S3 | 7.2 | 13.93 | 0.44 | 455.0373 | 455.0367 | -1.3 |
| Ceftiofur | C19H17N5O7S3 | 9.9 | 20.02 | 0.43 | 524.0363 | 524.0357 | -1.1 |
| Chloramphenicol | C11H12O5N2Cl2 | 12.7 | 18.06 | 0.44 | 323.0196 | 323.0204 | 2.5 |
| Chlorpyrifos ethyl | C9H11Cl3NO3PS | 7.4 | 31.55 | 0.34 | 349.9336 | 349.9344 | 2.3 |
| Chlortetracycline | C22H23ClN2O8 | 7.2 | 3.33 | 0.51 | 479.1216 | 479.121 | -1.2 |
| Ciprofloxacin | C17H18FN3O3 | 7.5 | 3.33 | 0.56 | 332.1405 | 332.1413 | 2.4 |
| Clenbuterol | C12H18Cl2N2O | 9.6 | 3.33 | 0.54 | 277.0869 | 277.0876 | 2.5 |
| Cloxacillin | C19H18ClN3O5S | 9.7 | 24.71 | 0.40 | 468.0991 | 468.0985 | -1.3 |
| Colchicine | C22H25NO6 | 9.4 | 19.65 | 0.45 | 400.1755 | 400.1749 | -1.5 |
| Cortisone | C21H28O5 | 12.7 | 20.95 | 0.44 | 361.201 | 361.2018 | 2.2 |
| Danofloxacin | C19H20FN3O3 | 8.0 | 3.34 | 0.55 | 358.1561 | 358.1569 | 2.2 |
| Dapsone | (H2NC6H4)2SO2 | 7.2 | 13.61 | 0.45 | 249.0692 | 249.0695 | 1.2 |
| Dicloxacillin | C19H17N3Cl2O5S | 7.5 | 26.16 | 0.37 | 470.0339 | 470.0333 | -1.3 |
| Difloxacin | C21H19F2N3O3 | 10.5 | 3.34 | 0.53 | 400.1467 | 400.1461 | -1.5 |
| Dimetridazole | C5H7N3O2 | 9.2 | 4.77 | 0.50 | 142.0611 | 142.0614 | 2.1 |
| Enoxacin | C15H17FN4O3 | 9.6 | 3.33 | 0.58 | 321.1357 | 321.1365 | 2.5 |
| Enrofloxacin | C19H22FN3O3 | 7.3 | 3.34 | 0.55 | 360.1718 | 360.1726 | 2.2 |
| Eprinomectin | C50H75NO14 | 9.2 | 23.33 | 0.48 | 914.526 | 914.5254 | -0.7 |
| Febantel | C20H22N4O6S | 11.7 | 13.06 | 0.51 | 331.1223 | 331.1231 | 2.4 |
| Febendazole | C15H13N3O2S | 7.6 | 23.32 | 0.41 | 300.0801 | 300.0799 | -0.7 |
| Fenthion | C10H15O3PS2 | 8.3 | 29.01 | 0.36 | 279.0273 | 279.028 | 2.5 |
| Fleroxacin | C17H18F3N3O3 | 7.6 | 3.33 | 0.56 | 370.1373 | 370.1381 | 2.2 |
| Flumequine | C14H12FNO3 | 7.0 | 22.49 | 0.41 | 262.0874 | 262.0881 | 2.7 |
| Imidacloprid | C9H10ClN5O2 | 7.4 | 13.86 | 0.45 | 256.0596 | 256.0603 | 2.7 |
| Ivermectin | C48H72O14 | 8.5 | 22.09 | 0.50 | 875.5151 | 875.5145 | -0.7 |
| Josamycin | C42H69NO15 | 8.0 | 17.03 | 0.65 | 828.474 | 828.4734 | -0.7 |
| Leucomalachite Green | C23H26N2 | 10.2 | 21.52 | 0.40 | 331.2169 | 331.2177 | 2.4 |
| Levamisole | C11H12N2S | 11.7 | 3.35 | 0.62 | 205.0794 | 205.0797 | 1.5 |
| Lincomycin | C18H35N2O6S | 7.2 | 3.33 | 0.63 | 407.221 | 407.2204 | -1.5 |
| Malachite Green | C23H24N2 | 10.2 | 20.91 | 0.47 | 329.2012 | 329.202 | 2.4 |
| Marbofloxacin | C17H20FN4O4 | 9.3 | 3.33 | 0.58 | 363.1463 | 363.1471 | 2.2 |
| Mebendazole | C16H13N3O3 | 8.1 | 21.11 | 0.43 | 296.103 | 296.1028 | -0.7 |
| Metronidazole | C6H9N3O3 | 6.8 | 3.56 | 0.54 | 128.0456 | 128.0459 | 2.3 |
| Minocycline | C23H28N3O7 | 11.1 | 3.34 | 0.67 | 458.1922 | 458.1916 | -1.3 |
| Nandrolon | C18H26O2 | 8.0 | 24.04 | 0.40 | 275.2006 | 275.2013 | 2.5 |
| Norfloxacin | C16H18FN3O3 | 6.5 | 3.33 | 0.47 | 320.1405 | 320.1413 | 2.5 |
| Ofloxacin | C18H20FN3O4 | 7.3 | 3.33 | 0.56 | 362.1511 | 362.1519 | 2.2 |
| Orbifloxacin | C19H21F3N3O3 | 8.7 | 3.33 | 0.54 | 396.153 | 396.1524 | -1.5 |
| Oxacillin | C19H19N3O5S | 7.0 | 23.89 | 0.41 | 434.138 | 434.1374 | -1.4 |
| Oxfendazole | C15H14N3O3S | 9.7 | 16.33 | 0.46 | 316.075 | 316.0758 | 2.5 |
| Oxolinic Acid | C13H11NO5 | 6.1 | 18.66 | 0.44 | 262.071 | 262.0717 | 2.7 |
| Oxybendazole | C12H15N3O3 | 7.7 | 20.14 | 0.38 | 250.1186 | 250.1189 | 1.2 |
| Oxytetracycline | C22H24N2O9 | 6.5 | 3.33 | 0.54 | 461.1555 | 461.1549 | -1.3 |
| Paclobutrazol | C15H20ClN3O | 10.7 | 25.54 | 0.39 | 294.1368 | 294.1366 | -0.7 |
| Penicillin G | C16H18N2O4S | 8.6 | 21.29 | 0.36 | 335.106 | 335.1068 | 2.4 |
| Penicillin V | C16H18N2O5S | 7.2 | 26.61 | 0.42 | 383.1271 | 383.1279 | 2.1 |
| Phenylbutazone | C19H20N2O2 | 7.4 | 29.85 | 0.37 | 309.1598 | 309.1606 | 2.6 |
| Ractopamine | C18H23O3 | 10.3 | 28.22 | 0.40 | 260.1645 | 260.1652 | 2.7 |
| Rifaximin | C43H51N3O11 | 9.0 | 28.75 | 0.39 | 786.3596 | 786.359 | -0.8 |
| Ronidazole | C6H8N4O4 | 9.5 | 22.05 | 0.51 | 140.0455 | 140.0458 | 2.1 |
| Roxithromycin | C41H76N2O15 | 11.2 | 20.39 | 0.55 | 837.5318 | 837.5312 | -0.7 |
| Sarafloxacin | C20H17F2N3O3 | 7.5 | 20.81 | 0.54 | 386.1311 | 386.1319 | 2.1 |
| Spinosad A | C41H65NO10 | 10.9 | 24.06 | 0.46 | 732.4681 | 732.4675 | -0.8 |
| Sulfabenzamide | C13H12N2O3S | 4.3 | 25.69 | 0.43 | 156.0114 | 156.0117 | 1.9 |
| Sulfacetamide | C8H10N2O3S | 10.0 | 21.89 | 0.51 | 156.0114 | 156.0117 | 1.9 |
| Sulfachloropyridazine | C10H11ClN4O2S | 11.4 | 24.73 | 0.45 | 285.0208 | 285.0206 | -0.7 |
| Sulfadiazine | C10H12N4O2S | 9.1 | 22.42 | 0.50 | 251.0597 | 251.06 | 1.2 |
| Sulfadimethoxyn | C12H14N4O4S | 7.5 | 25.73 | 0.43 | 311.0809 | 311.0817 | 2.6 |
| Sulfadoxine | C12H14N4O4S | 11.2 | 24.95 | 0.45 | 311.0809 | 311.0817 | 2.6 |
| Sulfaguanidine | C7H10N4O2S | 10.7 | 16.01 | 0.70 | 215.0597 | 215.06 | 1.4 |
| Sulfamerazine | C11H12N4O2S | 10.9 | 23.05 | 0.48 | 281.0703 | 281.071 | 2.5 |
| Sulfameter | C11H12N4O3S | 8.1 | 23.08 | 0.48 | 281.0703 | 281.0701 | -0.7 |
| Sulfamethazine | C12H14N4O2S | 7.6 | 23.86 | 0.47 | 279.091 | 279.0917 | 2.5 |
| Sulfamethoxazole | C10H13N3O3S | 7.6 | 24.99 | 0.45 | 254.0594 | 254.0597 | 1.2 |
| Sulfamethoxypyridazine | C11H12N4O3S | 9.0 | 22.31 | 0.50 | 281.0703 | 281.0701 | -0.7 |
| Sulfamonomethoxine | C11H12N4O3S | 9.6 | 22.33 | 0.50 | 281.0703 | 281.0701 | -0.7 |
| Sulfamoxole | C11H13N3O3S | 8.0 | 25.2 | 0.44 | 268.075 | 268.0757 | 2.6 |
| Sulfanitran | C14H13N3O5S | 9.2 | 26.51 | 0.42 | 336.0649 | 336.0657 | 2.4 |
| Sulfapyridine | C11H11N3O2S | 8.3 | 22.78 | 0.49 | 250.0645 | 250.0648 | 1.2 |
| Sulfaquinoxaline | C14H12N4O2S | 9.7 | 25.71 | 0.43 | 301.0754 | 301.0762 | 2.7 |
| Sulfathiazole | C9H11N3O2S2 | 9.7 | 22.4 | 0.50 | 256.0209 | 256.0216 | 2.7 |
| Sulfisoxazol | C11H13N3O3S | 5.7 | 23.21 | 0.48 | 268.075 | 268.0757 | 2.6 |
| Terbutaline hemisulfate | C12H19NO3 | 1.2 | 26.15 | 0.43 | 357.0955 | 357.0963 | 2.2 |
| Tetracycline | C22H24N2O8 | 9.3 | 20.74 | 0.54 | 445.1605 | 445.1599 | -1.4 |
| Thiabendazole | C10H7N3S | 8.9 | 19.89 | 0.56 | 202.0433 | 202.0436 | 1.5 |
| Tolfenamic Acid | C14H12ClNO2 | 9.3 | 31.39 | 0.36 | 262.0629 | 262.0636 | 2.7 |
| Trimethoprim | C14H20N4O3 | 7.1 | 19.01 | 0.59 | 291.1452 | 291.145 | -0.7 |
| a RSD calculated from 6 injections in different days (across 120 injections) in milk at 10 µg kg-1 diluted 100 times. |  |  |  |  |  |  |  |

Table 2. Recoveries (%) and linearity (R2) obtained in the studied matrices with the proposed nanoLC-HRMS method using a dilution factor of 100. Lowest concentration level assayed and the corresponding signal to noise are also included.

| Compound | Recovery (%) (n=6) |  |  |  |  |  |  |  | Lowest concentration level a |  |  |  |  |  |  |  | R2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Samples spiked at 10 µg kg-1 (dilution 1:100) |  |  |  |  |  |  |  | (µg kg-1) |  |  |  |  |  |  |  |  |  |  |  |
|  | Milk |  | Veal |  | Egg |  | Honey |  | Milk |  | Veal |  | Egg |  | Honey |  |  |  |  |  |
|  | R (%) | RSD (%) | R (%) | RSD (%) | R (%) | RSD (%) | R (%) | RSD (%) | Level | S/N | Level | S/N | Level | S/N | Level | S/N | Milk | Veal | Egg | Honey |
| Acetamiprid | 94.6 | 7.1 | 94.6 | 7.3 | 95.1 | 19.7 | 94.6 | 6.8 | 0.1 | 539 | 0.1 | 480 | 0.1 | 330 | 0.1 | 209 | 0.997 | 0.997 | 0.997 | 0.998 |
| Albendazole Sulfone | 93.9 | 18.3 | 93.9 | 18.3 | 93.9 | 10.3 | 93.9 | 18.3 | 1 | 1102 | 1 | 16 | 1 | 292 | 1 | 359 | 0.993 | 0.993 | 0.995 | 0.994 |
| Albendazole Sulfoxide | 94.2 | 12.8 | 94.2 | 19.7 | 94.2 | 18.3 | 94.2 | 9.7 | 1 | 45 | 1 | 497 | 1 | 456 | 1 | 156 | 0.999 | 0.999 | 0.992 | 0.995 |
| Alternogest | 112.3 | 2.4 | 112.3 | 2.5 | 112.3 | 11.2 | 112.3 | 8.4 | 0.1 | 69 | 0.1 | 16 | 1 | 186 | 0.1 | 48 | 0.999 | 0.999 | 0.992 | 0.997 |
| Amoxicillin | 107.9 | 19.7 | 107.9 | 17.9 | 107.9 | 9.5 | 108.0 | 20.1 | 1 | 19 | 0.1 | 16 | 1 | 19 | 1 | 60 | 0.995 | 0.995 | 0.995 | 0.996 |
| Betamethasone | 89.5 | 1.1 | 89.5 | 5.7 | 89.5 | 12.8 | 89.5 | 19.8 | 0.1 | 69 | 0.1 | 16 | 1 | 66 | 0.1 | 43 | 0.996 | 0.996 | 0.996 | 0.996 |
| Cefalexin | 102.2 | 11.2 | 102.2 | 17.7 | 102.2 | 7.7 | 102.2 | 8.5 | 0.1 | 654 | 0.1 | 215 | 0.1 | 215 | 0.1 | 241 | 0.994 | 0.994 | 0.994 | 0.994 |
| Cefazolin | 100.8 | 17.3 | 100.8 | 17.3 | 100.0 | 13.3 | 100.7 | 18.1 | 0.1 | 511 | 0.1 | 16 | 0.1 | 124 | 0.1 | 24 | 0.992 | 0.992 | 0.993 | 0.993 |
| Ceftiofur | 90.3 | 5.8 | 90.3 | 5.8 | 93.1 | 5.3 | 90.4 | 5.8 | 1 | 305 | 1 | 16 | 1 | 494 | 1 | 154 | 0.995 | 0.995 | 0.995 | 0.996 |
| Chloramphenicol | 109.2 | 14.2 | 109.2 | 14.2 | 109.2 | 3.9 | 109.4 | 14.2 | 0.1 | 230 | 0.1 | 193 | 0.1 | 180 | 0.1 | 128 | 0.998 | 0.998 | 0.998 | 0.998 |
| Chlorpyrifos | 98.5 | 20.0 | 98.5 | 17.8 | 98.5 | 4.6 | 98.6 | 9.7 | 1 | 2044 | 0.1 | 16 | 0.1 | 230 | 1 | 2057 | 0.998 | 0.998 | 0.998 | 0.999 |
| Chlortetra-cycline | 110.0 | 19.9 | 111.5 | 19.5 | 117.4 | 13.1 | 108.5 | 9.4 | 1 | 21 | 0.1 | 16 | 1 | 22 | 1 | 121 | 0.995 | 0.995 | 0.995 | 0.996 |
| Ciprofloxacin | 118.1 | 6.8 | 118.1 | 15.2 | 116.7 | 20.0 | 103.8 | 13.6 | 1 | 212 | 0.1 | 16 | 0.1 | 12 | 0.1 | 32 | 0.996 | 0.996 | 0.994 | 0.994 |
| Clenbuterol | 95.6 | 19.2 | 95.3 | 20.0 | 95.6 | 15.5 | 95.6 | 19.2 | 0.1 | 159 | 1 | 12 | 0.1 | 97 | 0.1 | 105 | 1 | 1 | 1 | 1 |
| Cloxacillin | 90.5 | 6.7 | 90.5 | 7.6 | 90.5 | 10.8 | 90.5 | 10.4 | 1 | 43 | 0.1 | 16 | 0.1 | 159 | 1 | 56 | 0.991 | 0.991 | 0.991 | 0.991 |
| Colchicine | 110.0 | 18.6 | 110.0 | 19.8 | 110.2 | 17.0 | 110.3 | 20.0 | 1 | 60 | 0.1 | 16 | 1 | 31 | 0.1 | 16 | 0.995 | 0.995 | 0.995 | 0.995 |
| Cortisone | 110.7 | 17.5 | 110.7 | 17.5 | 110.7 | 15.5 | 110.8 | 17.5 | 0.1 | 654 | 0.1 | 16 | 1 | 96 | 0.1 | 235 | 0.993 | 0.993 | 0.993 | 0.993 |
| Danofloxacin | 95.8 | 17.4 | 95.8 | 14.4 | 95.8 | 4.8 | 95.9 | 10.2 | 0.1 | 212 | 0.1 | 213 | 0.1 | 209 | 0.1 | 107 | 0.996 | 0.996 | 0.996 | 0.997 |
| Dapsone | 113.8 | 17.6 | 113.8 | 15.6 | 113.8 | 5.9 | 113.8 | 10.3 | 0.1 | 22 | 0.1 | 16 | 1 | 49 | 0.1 | 19 | 0.998 | 0.998 | 0.998 | 0.999 |
| Dicloxacillin | 95.0 | 19.5 | 95.0 | 15.9 | 95.0 | 19.2 | 95.0 | 7.7 | 0.1 | 116 | 1 | 66 | 1 | 87 | 0.1 | 77 | 0.993 | 0.993 | 0.993 | 0.993 |
| Difloxacin | 98.4 | 12.1 | 98.4 | 12.1 | 98.4 | 12.8 | 98.5 | 19.9 | 1 | 126 | 0.1 | 109 | 0.1 | 109 | 1 | 304 | 0.996 | 0.996 | 0.996 | 0.996 |
| Dimetridazole | 100.5 | 3.5 | 100.5 | 6.3 | 100.5 | 3.0 | 100.5 | 20.0 | 0.1 | 108 | 0.1 | 16 | 1 | 118 | 0.1 | 74 | 0.996 | 0.996 | 0.996 | 0.997 |
| Enoxacin | 92.5 | 3.0 | 92.5 | 3.1 | 92.5 | 3.0 | 92.5 | 3.0 | 1 | 53 | 0.1 | 71 | 0.1 | 78 | 1 | 18 | 0.99 | 0.99 | 0.99 | 0.995 |
| Enrofloxacin | 100.6 | 7.7 | 100.6 | 11.3 | 100.6 | 1.1 | 100.6 | 17.0 | 0.1 | 149 | 0.1 | 16 | 1 | 100 | 0.1 | 102 | 0.993 | 0.993 | 0.993 | 0.993 |
| Febantel | 95.8 | 14.1 | 95.8 | 16.6 | 95.8 | 10.5 | 95.8 | 9.5 | 0.1 | 149 | 0.1 | 16 | 1 | 60 | 0.1 | 101 | 0.995 | 0.995 | 0.995 | 0.996 |
| Febendazole | 114.8 | 3.0 | 114.8 | 3.0 | 114.8 | 5.8 | 114.8 | 0.0 | 0.1 | 219 | 1 | 4109 | 1 | 3657 | 0.1 | 124 | 1 | 1 | 1 | 1 |
| Fenthion | 110.1 | 5.7 | 110.1 | 5.7 | 100.1 | 15.0 | 110.1 | 5.7 | 1 | 14 | 0.1 | 16 | 0.1 | 12 | 1 | 3521 | 0.993 | 1 | 0.996 | 0.996 |
| Fleroxacin | 112.3 | 9.5 | 112.3 | 9.5 | 112.3 | 9.5 | 112.3 | 9.5 | 1 | 61 | 1 | 16 | 1 | 2578 | 1 | 357 | 0.996 | 0.996 | 0.996 | 0.996 |
| Flumequine | 115.7 | 17.4 | 115.7 | 19.5 | 115.7 | 10.5 | 115.7 | 9.8 | 1 | 34 | 0.1 | 16 | 1 | 14 | 1 | 89 | 0.994 | 0.994 | 0.994 | 0.994 |
| Imidacloprid | 120.5 | 8.5 | 120.5 | 10.2 | 120.0 | 8.5 | 120.5 | 10.5 | 0.1 | 46 | 0.1 | 154 | 0.1 | 156 | 0.1 | 25 | 0.999 | 1 | 1 | 0.999 |
| Josamycin | 104.5 | 10.8 | 104.5 | 9.6 | 104.5 | 5.7 | 104.5 | 6.6 | 0.1 | 508 | 0.1 | 105 | 0.1 | 92 | 0.1 | 199 | 0.997 | 0.997 | 0.997 | 0.997 |
| Leucomalachite Green | 104.5 | 18.6 | 104.5 | 19.8 | 104.5 | 18.6 | 104.5 | 5.4 | 0.1 | 55 | 0.1 | 180 | 0.1 | 193 | 0.1 | 38 | 0.995 | 0.995 | 0.995 | 0.995 |
| Levamisole | 103.3 | 15.7 | 103.3 | 15.7 | 103.3 | 13.0 | 103.3 | 10.0 | 0.1 | 51 | 0.1 | 16 | 1 | 105 | 0.1 | 26 | 0.995 | 0.995 | 0.995 | 0.996 |
| Lincomycin | 118.8 | 5.9 | 118.8 | 16.1 | 118.8 | 17.5 | 118.9 | 8.3 | 0.1 | 124 | 0.1 | 23 | 0.1 | 25 | 0.1 | 89 | 0.998 | 0.998 | 0.998 | 0.999 |
| Malachite Green | 88.5 | 4.8 | 88.5 | 4.8 | 88.5 | 4.8 | 88.5 | 4.8 | 1 | 93 | 1 | 105 | 1 | 93 | 1 | 58 | 0.991 | 0.991 | 0.991 | 0.991 |
| Marbofloxacin | 120.2 | 20.2 | 120.2 | 20.2 | 120.2 | 9.4 | 120.2 | 20.2 | 0.1 | 525 | 0.1 | 16 | 1 | 27 | 0.1 | 206 | 0.995 | 0.995 | 0.995 | 0.995 |
| Mebendazole | 102.7 | 7.2 | 102.7 | 13.0 | 102.7 | 5.3 | 102.8 | 9.6 | 0.1 | 4046 | 0.1 | 16 | 0.1 | 5387 | 0.1 | 589 | 0.991 | 0.991 | 0.994 | 0.992 |
| Metronidazole | 73.8 | 13.0 | 73.8 | 13.6 | 108.9 | 17.3 | 73.8 | 10.6 | 0.1 | 197 | 0.1 | 147 | 0.1 | 171 | 0.1 | 106 | 0.998 | 0.997 | 0.999 | 0.995 |
| Minocycline | 118.0 | 9.8 | 118.3 | 18.2 | 118.0 | 3.5 | 118.0 | 5.4 | 0.1 | 221 | 0.1 | 201 | 0.1 | 48 | 0.1 | 127 | 0.999 | 0.999 | 0.999 | 0.999 |
| Nandrolon | 109.1 | 7.6 | 109.1 | 6.7 | 109.1 | 17.6 | 109.1 | 5.4 | 1 | 30 | 0.1 | 201 | 0.1 | 205 | 1 | 43 | 0.998 | 0.998 | 0.998 | 0.998 |
| Norfloxacin | 91.2 | 18.9 | 91.2 | 18.9 | 91.2 | 14.7 | 91.3 | 18.8 | 1 | 18 | 0.1 | 16 | 1 | 81 | 1 | 74 | 0.997 | 0.997 | 0.997 | 0.997 |
| Ofloxacin | 86.9 | 16.5 | 86.9 | 16.7 | 86.9 | 6.8 | 86.9 | 20.0 | 0.1 | 1363 | 0.1 | 103 | 0.1 | 87 | 0.1 | 402 | 0.998 | 0.990 | 0.993 | 0.991 |
| Orbifloxacin | 113.7 | 10.5 | 113.6 | 13.2 | 109.8 | 8.9 | 113.7 | 10.5 | 0.1 | 604 | 0.1 | 154 | 0.1 | 156 | 0.1 | 214 | 0.994 | 0.995 | 0.995 | 0.995 |
| Oxacillin | 105.7 | 4.6 | 105.7 | 8.7 | 105.7 | 15.7 | 105.8 | 10.3 | 0.1 | 1027 | 0.1 | 248 | 0.1 | 248 | 0.1 | 258 | 0.995 | 0.995 | 0.995 | 0.995 |
| Oxfendazole | 113.7 | 5.3 | 113.7 | 5.3 | 119.0 | 8.4 | 113.7 | 5.3 | 0.1 | 573 | 0.1 | 16 | 1 | 52 | 0.1 | 214 | 0.994 | 0.994 | 0.994 | 0.994 |
| Oxolinic Acid | 101.1 | 13.4 | 101.1 | 14.7 | 101.1 | 14.1 | 101.1 | 6.8 | 0.1 | 17 | 0.1 | 398 | 0.1 | 400 | 0.1 | 16 | 0.993 | 0.994 | 0.994 | 0.994 |
| Oxybendazole | 120.3 | 10.5 | 120.3 | 9.3 | 120.3 | 12.1 | 120.2 | 5.8 | 1 | 105 | 0.1 | 51 | 0.1 | 49 | 1 | 118 | 0.994 | 0.994 | 0.994 | 0.994 |
| Oxytetracycline | 116.3 | 9.4 | 116.3 | 5.9 | 116.3 | 19.5 | 116.2 | 6.8 | 0.1 | 37 | 0.1 | 16 | 1 | 3495 | 0.1 | 21 | 0.995 | 0.994 | 0.995 | 0.995 |
| Paclobutrazol | 111.2 | 7.4 | 111.2 | 7.4 | 111.2 | 7.4 | 111.1 | 7.4 | 1 | 54 | 0.1 | 16 | 1 | 325 | 1 | 63 | 0.997 | 0.997 | 0.997 | 0.998 |
| Penicillin G | 93.7 | 13.8 | 93.7 | 13.8 | 93.7 | 12.1 | 93.7 | 13.8 | 0.1 | 360 | 0.1 | 16 | 1 | 30 | 0.1 | 181 | 0.99 | 0.99 | 0.993 | 0.991 |
| Penicillin V | 95.3 | 15.5 | 95.3 | 15.5 | 95.3 | 3.0 | 95.3 | 17.9 | 0.1 | 19 | 0.1 | 16 | 1 | 56 | 0.1 | 17 | 0.991 | 0.991 | 0.994 | 0.992 |
| Phenylbutazone | 94.2 | 14.7 | 94.2 | 14.7 | 95.5 | 13.7 | 94.1 | 14.7 | 0.1 | 58079 | 0.1 | 36 | 0.1 | 26 | 0.1 | 856 | 0.99 | 0.99 | 0.993 | 0.991 |
| Ractopamine | 91.3 | 10.3 | 91.3 | 10.3 | 91.3 | 4.4 | 91.3 | 10.3 | 1 | 201 | 0.1 | 39722 | 0.1 | 38387 | 1 | 214 | 0.993 | 0.993 | 0.995 | 0.994 |
| Rifaximin | 104.8 | 15.5 | 104.8 | 15.5 | 104.8 | 14.2 | 104.8 | 16.9 | 0.1 | 97 | 0.1 | 16 | 1 | 98 | 0.1 | 59 | 0.998 | 0.998 | 0.998 | 0.999 |
| Ronidazole | 116.1 | 13.1 | 116.1 | 13.1 | 116.1 | 12.3 | 116.1 | 13.1 | 0.1 | 265 | 0.1 | 23 | 0.1 | 43 | 0.1 | 148 | 0.999 | 0.999 | 0.992 | 0.996 |
| Roxithromycin | 120.1 | 13.7 | 120.1 | 13.7 | 120.1 | 19.9 | 120.1 | 10.1 | 0.1 | 37 | 0.1 | 16 | 1 | 8530 | 0.1 | 19 | 0.997 | 0.997 | 0.997 | 0.998 |
| Sarafloxacin | 113.9 | 3.9 | 113.9 | 3.9 | 113.9 | 18.6 | 113.9 | 10.4 | 0.1 | 71 | 0.1 | 144 | 0.1 | 144 | 0.1 | 53 | 0.993 | 0.993 | 0.993 | 0.993 |
| Spinosad A | 96.9 | 12.1 | 96.9 | 12.1 | 96.9 | 10.1 | 96.9 | 12.1 | 1 | 169 | 0.1 | 102 | 0.1 | 86 | 1 | 182 | 0.993 | 0.993 | 0.995 | 0.994 |
| Sulfabenzamide | 91.2 | 12.3 | 91.2 | 12.3 | 91.2 | 19.9 | 91.1 | 12.4 | 0.1 | 377 | 0.1 | 2017 | 0.1 | 2119 | 0.1 | 187 | 0.993 | 0.994 | 0.993 | 0.993 |
| Sulfacetamide | 115.5 | 19.9 | 115.5 | 19.9 | 115.5 | 16.7 | 115.5 | 9.6 | 0.1 | 117 | 0.1 | 231 | 0.1 | 225 | 0.1 | 81 | 0.994 | 0.994 | 0.994 | 0.995 |
| Sulfachloropyri-dazine | 97.1 | 10.1 | 97.1 | 19.5 | 97.1 | 20.2 | 97.1 | 10.2 | 1 | 24 | 0.1 | 16 | 0.1 | 352 | 1 | 137 | 0.997 | 0.995 | 0.996 | 0.996 |
| Sulfadiazine | 94.3 | 4.4 | 94.3 | 4.4 | 94.3 | 4.4 | 94.2 | 4.4 | 1 | 26 | 0.1 | 16 | 1 | 29 | 1 | 1547 | 0.996 | 0.996 | 0.996 | 0.997 |
| Sulfadimethoxyn | 115.0 | 17.2 | 115.0 | 17.0 | 115.0 | 7.6 | 115.0 | 9.9 | 1 | 33 | 0.1 | 178 | 0.1 | 154 | 1 | 86 | 0.994 | 0.994 | 0.994 | 0.994 |
| Sulfadoxine | 120.4 | 8.4 | 120.4 | 8.4 | 120.4 | 8.4 | 120.4 | 8.5 | 0.1 | 796 | 0.1 | 16 | 1 | 64 | 0.1 | 241 | 0.995 | 0.995 | 0.995 | 0.996 |
| Sulfaguanidine | 118.7 | 2.0 | 118.7 | 10.0 | 118.7 | 19.0 | 118.7 | 14.0 | 1 | 12 | 0.1 | 16 | 1 | 72 | 1 | 129 | 0.997 | 0.997 | 0.997 | 0.998 |
| Sulfamerazine | 90.5 | 19.6 | 90.5 | 16.9 | 90.5 | 16.5 | 90.4 | 9.8 | 1 | 3019 | 0.1 | 16 | 1 | 2888 | 0.1 | 12 | 0.998 | 0.998 | 0.998 | 0.999 |
| Sulfameter | 110.6 | 10.3 | 110.6 | 6.8 | 110.6 | 2.4 | 110.5 | 19.5 | 0.1 | 15 | 0.1 | 16 | 0.1 | 576 | 0.1 | 15 | 0.999 | 0.999 | 0.992 | 0.99 |
| Sulfamethazine | 107.2 | 10.1 | 107.2 | 10.1 | 107.2 | 17.2 | 107.2 | 10.1 | 1 | 1102 | 0.1 | 16 | 1 | 971 | 0.1 | 12 | 0.999 | 0.999 | 0.999 | 1 |
| Sulfamethoxa-zole | 90.6 | 17.3 | 90.6 | 20.0 | 90.6 | 18.9 | 90.2 | 9.9 | 1 | 202 | 0.1 | 16 | 1 | 71 | 0.1 | 12 | 0.998 | 0.998 | 0.998 | 0.999 |
| Sulfamethoxy-pyridazine | 120.1 | 7.7 | 120.1 | 7.7 | 120.1 | 7.7 | 120.1 | 7.7 | 0.1 | 1098 | 0.1 | 16 | 1 | 71 | 0.1 | 301 | 0.994 | 0.994 | 0.994 | 0.995 |
| Sulfamonome-thoxine | 103.7 | 8.2 | 103.7 | 16.2 | 103.7 | 18.9 | 103.7 | 10.1 | 0.1 | 89 | 0.1 | 54 | 0.1 | 36 | 0.1 | 54 | 0.996 | 0.996 | 0.996 | 0.997 |
| Sulfamoxole | 105.3 | 5.4 | 105.3 | 5.4 | 105.3 | 5.4 | 105.3 | 5.4 | 0.1 | 99 | 0.1 | 18 | 0.1 | 14 | 0.1 | 60 | 0.997 | 0.997 | 0.997 | 0.998 |
| Sulfanitran | 114.7 | 8.4 | 114.7 | 8.3 | 114.7 | 8.4 | 114.6 | 8.3 | 0.1 | 322 | 0.1 | 16 | 1 | 80 | 0.1 | 158 | 0.994 | 0.994 | 0.994 | 0.994 |
| Sulfapyridine | 102.9 | 15.8 | 102.9 | 19.8 | 102.9 | 6.7 | 102.9 | 16.2 | 1 | 23 | 0.1 | 64 | 0.1 | 64 | 0.1 | 12 | 0.999 | 0.999 | 0.999 | 1 |
| Sulfaquinoxaline | 96.3 | 5.4 | 96.3 | 5.4 | 96.3 | 5.4 | 96.3 | 5.4 | 0.1 | 57 | 0.1 | 68 | 0.1 | 61 | 0.1 | 43 | 0.994 | 0.994 | 0.994 | 0.995 |
| Sulfathiazole | 101.2 | 6.8 | 101.2 | 6.8 | 101.2 | 6.8 | 101.2 | 6.8 | 0.1 | 101 | 0.1 | 1578 | 0.1 | 1896 | 0.1 | 63 | 0.995 | 0.995 | 0.995 | 0.996 |
| Sulfisoxazol | 94.6 | 5.8 | 94.6 | 5.8 | 94.6 | 5.8 | 94.6 | 5.8 | 0.1 | 13 | 0.1 | 48 | 0.1 | 40 | 0.1 | 14 | 0.999 | 0.999 | 0.999 | 0.999 |
| Terbutaline hemisulfate | 112.4 | 5.4 | 112.4 | 5.4 | 112.4 | 5.4 | 112.3 | 5.4 | 0.1 | 52 | 0.1 | 66 | 0.1 | 67 | 0.1 | 34 | 0.991 | 0.991 | 0.994 | 0.992 |
| Tetracycline | 105.5 | 20.0 | 105.5 | 20.0 | 105.5 | 13.8 | 105.5 | 20.0 | 0.1 | 104 | 0.1 | 90 | 0.1 | 79 | 0.1 | 68 | 0.993 | 0.993 | 0.995 | 0.994 |
| Thiabendazole | 107.1 | 8.5 | 107.1 | 8.5 | 107.1 | 8.5 | 107.1 | 8.5 | 0.1 | 13 | 0.1 | 110 | 0.1 | 110 | 1 | 124 | 0.997 | 0.997 | 0.997 | 0.998 |
| Tolfenamic Acid | 90.5 | 4.7 | 90.5 | 6.8 | 90.5 | 9.8 | 90.4 | 10.0 | 1 | 11 | 0.1 | 105 | 0.1 | 93 | 0.1 | 12 | 0.991 | 0.991 | 0.991 | 0.991 |
| Trimethoprim | 82.3 | 11.4 | 102.9 | 17.9 | 102.9 | 4.5 | 111.8 | 15.1 | 0.1 | 13 | 0.1 | 31 | 0.1 | 37 | 0.1 | 13 | 0.999 | 0.999 | 0.999 | 1 |
| Acriflavine | 98.2 | 6.6 | 98.2 | 6.6 | 98.2 | 6.6 | 98.2 | 6.5 | 0.1 | 873 | 0.1 | 48 | 0.1 | 35 | 0.1 | 248 | 0.993 | 0.993 | 0.993 | 0.993 |
| Albendazole | 102.1 | 20.1 | 102.1 | 20.1 | 102.1 | 7.4 | 101.8 | 20.0 | 1 | 13 | 0.1 | 78 | 0.1 | 78 | 0.1 | 12 | 0.994 | 0.994 | 0.994 | 0.994 |
| Carbosulfan | 120.0 | 3.7 | 120.0 | 3.7 | 120.0 | 3.7 | 120.0 | 3.7 | 0.1 | 407 | 0.1 | 21 | 0.1 | 18 | 0.1 | 189 | 0.994 | 0.994 | 0.994 | 0.995 |
| Eprinomectin | 117.1 | 17.6 | 117.1 | 16.7 | 117.1 | 17.4 | 117.1 | 10.6 | 1 | 2922 | 0.1 | 16 | 0.1 | 1010 | 0.1 | 12 | 0.999 | 1 | 0.999 | 0.999 |
| Ivermectin | 93.3 | 5.5 | 93.3 | 5.5 | 93.3 | 19.7 | 93.3 | 4.4 | 0.1 | 134 | 0.1 | 1000 | 0.1 | 315 | 0.1 | 100 | 0.991 | 0.996 | 0.994 | 0.994 |

a Corresponds to the lowest concentration level studied yielding S/N ratios equal or higher than 10.

Table 3. Matrix effects (%) obtained in the studied matrices with the proposed nanoLC-HRMS method using a dilution factor of 100.

| Compound | Matrix Effects (%) a |  |  |  |
| --- | --- | --- | --- | --- |
|  | Milk | Veal | Egg | Honey |
| Acetamiprid | 0.0 | 0.0 | 0.0 | 0.1 |
| Albendazole Sulfone | 0.1 | 0.7 | 0.1 | 1.5 |
| Albendazole Sulfoxide | 0.1 | 1.0 | 0.1 | 1.5 |
| Alternogest | 0.0 | 0.2 | 0.0 | 0.5 |
| Amoxicillin | 0.0 | 0.6 | 0.0 | 1.4 |
| Betamethasone | 0.0 | 0.0 | 0.0 | 0.1 |
| Cefalexin | 0.3 | 1.3 | 0.2 | 2.0 |
| Cefazolin | 0.0 | 0.7 | 0.9 | 0.8 |
| Ceftiofur | 0.1 | 1.2 | 0.1 | 2.6 |
| Chloramphenicol | 0.2 | 6.3 | 0.7 | 9.4 |
| Chlorpyrifos | 0.1 | 1.4 | 0.2 | 2.1 |
| Chlortetra-cycline | 0.0 | 0.6 | 0.1 | 0.9 |
| Ciprofloxacin | 0.0 | 0.8 | 1.4 | 0.3 |
| Clenbuterol | 0.1 | 1.2 | 0.1 | 1.1 |
| Cloxacillin | 0.0 | 0.1 | 0.0 | 0.1 |
| Colchicine | 0.0 | 0.4 | 0.4 | 0.8 |
| Cortisone | 1.2 | 1.3 | 1.3 | 2.0 |
| Danofloxacin | 0.0 | 1.0 | 0.1 | 1.4 |
| Dapsone | 0.0 | 0.9 | 0.1 | 1.4 |
| Dicloxacillin | 0.0 | 0.9 | 0.1 | 1.3 |
| Difloxacin | 0.1 | 1.5 | 0.2 | 2.3 |
| Dimetridazole | 0.0 | 1.1 | 0.1 | 2.4 |
| Enoxacin | 0.1 | 1.2 | 0.1 | 1.7 |
| Enrofloxacin | 0.5 | 1.2 | 0.6 | 1.7 |
| Febantel | 0.0 | 0.6 | 0.0 | 1.3 |
| Febendazole | 0.0 | 0.5 | 0.0 | 1.1 |
| Fenthion | 1.7 | 1.1 | 0.3 | 1.7 |
| Fleroxacin | 0.0 | 0.2 | 0.0 | 0.5 |
| Flumequine | 0.0 | 0.1 | 0.0 | 0.3 |
| Imidacloprid | 0.0 | 0.0 | 0.0 | 0.0 |
| Josamycin | 0.0 | 0.2 | 0.0 | 0.5 |
| Leucomalachite Green | 0.0 | 1.4 | 0.1 | 2.1 |
| Levamisole | 0.0 | 0.2 | 0.0 | 0.3 |
| Lincomycin | 0.2 | 3.6 | 0.4 | 5.4 |
| Malachite Green | 0.1 | 1.2 | 0.1 | 1.8 |
| Marbofloxacin | 0.0 | 0.1 | 0.0 | 0.3 |
| Mebendazole | 0.5 | 1.9 | 0.4 | 3.0 |
| Metronidazole | 0.2 | 0.0 | 0.2 | 0.0 |
| Minocycline | 0.2 | 0.8 | 0.2 | 1.2 |
| Nandrolon | 0.0 | 0.9 | 0.0 | 1.9 |
| Norfloxacin | 0.4 | 1.2 | 0.3 | 1.8 |
| Ofloxacin | 0.0 | 0.1 | 0.0 | 0.3 |
| Orbifloxacin | 0.2 | 0.2 | 0.6 | 1.3 |
| Oxacillin | 0.2 | 4.6 | 0.6 | 6.8 |
| Oxfendazole | 0.0 | 0.1 | 0.0 | 0.1 |
| Oxolinic Acid | 0.1 | 1.3 | 0.2 | 2.0 |
| Oxybendazole | 0.1 | 1.2 | 0.2 | 1.9 |
| Oxytetracycline | 0.1 | 1.3 | 0.2 | 1.9 |
| Paclobutrazol | 0.1 | 1.1 | 0.1 | 1.7 |
| Penicillin G | 0.0 | 0.1 | 0.0 | 0.3 |
| Penicillin V | 0.0 | 1.2 | 0.1 | 2.6 |
| Phenylbutazone | 0.0 | 0.1 | 0.0 | 0.3 |
| Ractopamine | 0.0 | 0.7 | 0.0 | 1.5 |
| Rifaximin | 0.0 | 0.0 | 0.0 | 0.0 |
| Ronidazole | 0.1 | 1.6 | 0.2 | 2.4 |
| Roxithromycin | 0.0 | 0.2 | 0.0 | 0.5 |
| Sarafloxacin | 0.0 | 0.1 | 0.0 | 0.2 |
| Spinosad A | 0.2 | 0.6 | 0.1 | 0.9 |
| Sulfabenzamide | 1.9 | 4.0 | 1.6 | 6.0 |
| Sulfacetamide | 0.0 | 0.0 | 0.0 | 0.1 |
| Sulfachloropyri-dazine | 1.7 | 1.5 | 1.6 | 2.3 |
| Sulfadiazine | 0.1 | 1.1 | 0.2 | 2.4 |
| Sulfadimethoxyn | 0.0 | 1.2 | 0.1 | 2.5 |
| Sulfadoxine | 0.0 | 0.9 | 0.1 | 1.3 |
| Sulfaguanidine | 0.0 | 0.4 | 0.0 | 0.8 |
| Sulfamerazine | 0.1 | 1.4 | 0.2 | 2.1 |
| Sulfameter | 0.1 | 1.2 | 0.1 | 1.9 |
| Sulfamethazine | 0.0 | 0.6 | 0.1 | 0.9 |
| Sulfamethoxa-zole | 0.5 | 9.2 | 1.1 | 3.8 |
| Sulfamethoxy-pyridazine | 0.0 | 0.3 | 0.0 | 0.6 |
| Sulfamonome-thoxine | 0.0 | 0.6 | 0.1 | 0.9 |
| Sulfamoxole | 0.0 | 1.0 | 0.1 | 1.4 |
| Sulfanitran | 0.1 | 1.0 | 0.2 | 1.5 |
| Sulfapyridine | 0.0 | 0.8 | 0.1 | 1.2 |
| Sulfaquinoxaline | 0.0 | 0.3 | 0.0 | 0.7 |
| Sulfathiazole | 0.0 | 1.2 | 0.1 | 2.6 |
| Sulfisoxazol | 0.0 | 1.2 | 0.1 | 2.7 |
| Terbutaline hemisulfate | 0.1 | 2.0 | 0.2 | 3.0 |
| Tetracycline | 0.0 | 0.6 | 0.1 | 0.9 |
| Thiabendazole | 0.0 | 0.7 | 0.1 | 1.1 |
| Tolfenamic Acid | 0.1 | 1.4 | 0.2 | 2.1 |
| Trimethoprim | 0.0 | 0.6 | 0.0 | 1.3 |
| Acriflavine | 0.0 | 0.9 | 0.0 | 1.9 |
| Albendazole | 0.7 | 6.4 | 0.2 | 9.6 |
| Carbosulfan | 0.0 | 0.0 | 0.0 | 0.1 |
| Eprinomectin | 0.0 | 1.0 | 0.1 | 2.3 |
| Ivermectin | 0.0 | 0.1 | 0.0 | 0.3 |

a Matrix effects calculated as [(calibration curve slope in matrix/calibration curve slope in solvent)-1] x 100.

Table 4. Veterinary drugs found in the study of real samples of milk and MRLs established for each compound found in milk matrix.

|  | Oxacyllin | Oxytetracycline | Penicillin G | Penicillin V | Thiabendazole |
| --- | --- | --- | --- | --- | --- |
| MRLa (g kg-1) | 30 | 100 | Not legislated | 4 | 100 |
| Sample 1 | 0.52 | 0.45 | < 0.1b | - | 0.13 |
| Sample 2 | 0.34 | 0.87 | < 0.1b | - | 0.15 |
| Sample 3 | 0.30 | - | < 0.1b | < 0.1b | 0.28 |
| Sample 4 | - | - | < 0.1b | < 0.1b | 0.10 |

a European Commission Regulation (EU) No 37/2010 (European Commission, 2010).

b Result below the lowest concentration level assayed.

References

Aguilera-Luiz, M.M., Martínez Vidal, J.L., Romero-González, R., & Garrido-Frenich, A. (2008) Multi-residue determination of veterinary drugs in milk by ultra-high-pressure liquid chromagraphy-tandem mass spectrometry. Journal of Chromatography A, 1205, 10-16.

Bargańska, Ż., Namieśnik, J., & Ślebioda, M. (2011). Determination of antibiotic residues in honey. TrAC Trends Analytical Chemistry, 30, 1035-1041.

Cappiello, A., Famiglini, G., Mangani, F., Palma, P., & Siviero, A. (2003). Nanohigh-performance liquid chromatography–electron ionization mass spectrometry approach for environmental analysis. Analytica Chimica Acta, 493, 125-136.

Cepurnieks, G., Rjabova, J., Zacs, D., & Bartkevics, V. (2015). The development and validation of a rapid method for the determination of antimicrobial agent residues in milk and meat using ultra performance liquid chromatography coupled to quadrupole – Orbitrap mass spectrometry. Journal of Pharmaceutical Biomedical, 102, 184-192.

Chambers, E., Wagrowski-Diehl, D.M., Lu, Z., & Mazzeo, J.R. (2007). Systematic and comprehensive strategy for reducing matrix effects in LC/MS/MS analyses. Journal of Chromatography B, 852, 22-34.

Dasenaki, M.E., Michali, C.S., & Thomaidis, N.S. (2016). Analysis of 76 veterinary pharmaceuticals from 13 classes including aminoglycosides in bovine muscle by hydrophilic interaction liquid chromatography–tandem mass spectrometry. Journal of Chromatography A, 1452, 67-80.

Desmet, G., & Eeltink, S. (2013). Fundamentals for LC Miniaturization. Analytical Chemistry, 85, 543-556.

European Commission (2002). European Commission decision of 12 August 2002 implementing Council Directive 96/23/EC concerning the performance of analytical methods and the interpretation of results, Commission Decision 2002/657/EEC. The Official Journal of the European Union, L221, 8-36.

European Commission. (2010). Commission Regulation (EU) 37/2010, on pharmacologically active substances and their classification regarding maximum residue limits in foodstuffs of animal origin. The Official Journal of the European Union, L15, 1-72.

European Commission. (2015). Guidance document on analytical quality control and method validation procedures for pesticides residues analysis in food and feed. SANTE/11945/2015.

Florea, N.F., & Nightingale, C.H. (2004). Review of the pharmacodynamics of antibiotic use in animal food production. Diagnostic Microbiology and Infectious Disease, 49, 105-108.

Food and Agriculture Organization (FAO) (2003). http://www.fao.org/ag/againfo/home/en/news_archive/2011_the_roles_of_veterinarians.html  Accessed 12.07.17.

Food and Agriculture Organization (FAO). (2015). Maximum residue limits (MRLs) and risk management recommendations (RMRs) for residues of veterinary drugs in foods. http://www.fao.org/input/download/standards/45/MRL2_2015e.pdf

Fu, L., Amato, N.J., Wang, P., McGowan, S.J., Niedernhofer, L.J., & Wang, Y. (2015). Simultaneous Quantification of Methylated Cytidine and Adenosine in Cellular and Tissue RNA by Nano-Flow Liquid Chromatography-Tandem Mass Spectrometry Coupled with the Stable Isotope-dilution Method. Analytical Chemistry, 87, 7653-7659.

Garrido Frenich, A., Aguilera-Luiz, M.M., Martinez Vidal, J.L., & Romero-Gonzalez, R. (2010). Comparison of several extraction techniques for multiclass analysis of veterinary drugs in eggs using ultra-high pressure liquid chromatography-tandem mass spectrometry. Analytica Chimica Acta, 661, 150-160.

Garrido Frenich, A., Romero-González, R., & Aguilera-Luiz, M.M. (2014). Comprehensive analysis of toxics (pesticides, veterinary drugs and mycotoxins) in food by UHPLC-MS. Trends in Analytical Chemistry, 63, 158-169.

Geis-Asteggiante, L., Lehotay, S.J., & Lightfield, A.R. (2012). Ruggedness testing and validation of a practical analytical method for >100 veterinary drug residues in bovine muscle by ultrahigh performance liquid chromatography-tandem mass spectrometry. Journal of Chromatography A, 1258, 43-54.

González-Antuña, A., Domínguez-Romero, J.C., García-Reyes, J.F., Rodríguez-González, P., Centineo, G., García-Alonso, J.I., Molina-Díaz, A. (2013). Overcoming matrix effects in electrospray: Quantitation of β-agonists in complex matrices by isotope dilution liquid chromatography–mass spectrometry using singly 13C-labeled analogues. Journal of Chromatography A, 1288, 40-47.

Gómez-Pérez, M.L., Plaza-Bolanos, P., Romero-González, R., Martínez-Vidal, J.L., & Garrido-Frenich, A. (2012). Comprehensive qualitative and quantitative determination of pesticides and veterinary drugs in honey using liquid chromatography–Orbitrap high resolution mass spectrometry. Journal of Chromatography. A, 1248, 130-138.

Kaufmann, A., Butcher, P., Maden, K., Walker, S., & Widmer, M. (2014). Multi-residue quantification of veterinary drugs in milk with a novel extraction and cleanup technique: Salting out supported liquid extraction (SOSLE). Analytica Chimica Acta, 820, 56-68.

Kebarle, P., & Tang, L. (1993). From ions in solution to ions in the gas phase - the mechanism of electrospray mass spectrometry. Analytical Chemistry, 65, 972A-986A.

Kiyonami, R., Ravnsborg, C., Madsen, O., & Zabrouskov, V. (2012). Easy-to-Use, Plug-and-Spray Ion Source for Robust and Reproducible Ultra High Pressure Nanoflow LC/MS. In Technical Note 63546. Thermo Fisher Scientific San Jose, USA.

Landers, T.F., Cohen, B., Wittum, T.E., & Larson, E.L.  (2012). A Review of Antibiotic Use in Food Animals: Perspective, Policy, and Potential. Public Health Reports, 127, 4-22.

Masiá, A., Suarez-Varela, M.M., Llopis-González, A., & Picó, Y. (2016). Determination of pesticides and veterinary drug residues in food by liquid chromatography-mass spectrometry: A review. Analytica Chimica Acta, 936, 40-61.

Matuszewski, B.K., Constanzer, M.L., & Chavez-Eng C.M. (2003). Strategies for the Assessment of Matrix Effect in Quantitative Bioanalytical Methods Based on HPLC-MS/MS. Analytical Chemistry, 75, 3019-3030.

Matuszewski, B.K. (2006). Standard line slopes as a measure of a relative matrix effect in quantitative HPLC-MS. Bioanalysis, 830, 293-300.

Meher, A.K., & Chen, Y.C. (2017). Electrospray Modifications for Advancing Mass Spectrometric Analysis. Mass spectrometry, 6, S0057.

Moreno-González, D., & García-Campaña A.M. (2017). Salting-out assisted liquid-liquid extraction coupled to ultra-high performance liquid chromatography-tandem mass spectrometry for the determination of tetracycline residues in infant food. Food Chemistry, 221, 1763-1769.

Ortelli, D., Cognard, E., Jan, P., & Edder, P. (2009). Comprehensive fast multiresidue screening of 150 veterinary drugs in milk by ultraperformance liquid chromatography coupled to time of flight mass spectrometry. Journal of Chromatography B, 877, 2363-2374.

Peters, R.J.B., Bolck, Y.J.C., & Rutgers, P. (2009). Multi-residue screening of veterinary drugs in egg, fish and meat using high-resolution liquid chromatography accurate mass time-of-flight mass spectrometry. Journal of Chromatography A, 1216, 8206-8216.

Pinto da Costa, R., Ferraz Spisso, B., Ulberg Pereira M., Alves Monteiro, M., Gomes Ferreira R., & da Nóbrega, A. W. (2015). Innovative mixture of salts in the quick, easy, cheap, effective, rugged, and safe method for the extraction of residual macrolides in milk followed by analysis with liquid chromatography and tandem mass spectrometry. Journal of Separation Sciences, 38, 3743-3749.

Rajski, L., Gómez-Ramos, M.M, & Fernández Alba, A. R. (2014). Large pesticide multiresidue screening method by liquid chromatography-Orbitrap mass spectrometry in full scan mode applied to fruit and vegetables. Journal of Chromatography A, 1360, 119-127.

Robert, C., Gillard, N., & Brasseur, P.Y. (2013). Rapid multi-residue and multi-class qualitative screening for veterinary drugs in foods of animal origin by UHPLC-MS/MS. Food Additives and Contaminants Part A, 30, 443-457.

Schmidt, A., Karas, M., & Dülcks, T. (2003). Effect of different solution flow rates on analyte ion signals in nano-ESI MS, or: When does ESI turn into nano-ESI? Journal of the American Society for Mass Spectrometry, 14, 492-500.

Shen, Y., Zhao, R., Berger, S.J., Anderson, G.A., Rodriguez, N., & Smith, R.D. (2002). High-Efficiency Nanoscale Liquid Chromatography Coupled On-Line with Mass Spectrometry Using Nanoelectrospray Ionization for Proteomics. Analytical Chemistry, 74, 423-4249.

Stahnke, H., Kittlaus, S., Kempe, G., & Alder, L. (2012). Reduction of Matrix Effects in Liquid Chromatography–Electrospray Ionization–Mass Spectrometry by Dilution of the Sample Extracts: How Much Dilution is Needed? Analytical Chemistry 84, 1474-1482.

Trufelli, H., Palma, P., Famiglini, G., &  Cappiello, A. (2011). An overview of matrix effects in liquid chromatography–mass spectrometry. Mass Spectrometry Reviews 30, 491-509.

Wang, J., Leung, D., Chow, W., Chang, J., & Wong, J.W. (2015). Development and Validation of a Multiclass Method for Analysis of Veterinary Drug Residues in Milk Using Ultrahigh Performance Liquid Chromatography Electrospray Ionization Quadrupole Orbitrap Mass Spectrometry. Journal of Agriculture and Food Chemistry, 63, 9175-9187.

Wei, Y., Zhang, Y., Xu, J., Gou, C., Li, L., & Fan, W. (2014).  Simultaneous quantification of several classes of antibiotics in water, sediments, and fish muscles by liquid chromatography-tandem mass spectrometry. Frontiers of Environmental Science & Engineering, 8, 357-371.

Wilm, M., & Mann, M. (1996). Analytical Properties of the Nanoelectrospray Ion Source. Analytical Chemistry, 68, 1-8.

Wilson, S.R., Vehus, T., Berg, H.S., & Lundanes, E. (2015).  Nano-LC in proteomics: recent advances and approaches. Bioanalysis 7, 1799-1815.